Утвержден МЦКТ.002.021 ПО ТП-ЛУ

ЧИСЛЕННАЯ МОДЕЛЬ КВАНТОВОЙ ИНТЕГРАЛЬНОЙ МИКРОСХЕМЫ ИЗ НЕ МЕНЕЕ 16 КУБИТОВ

Текст программы МЦКТ.002.021 ПО ТП

Листов 55

20___ г.

Москва

| СОГЛАСОВАНО Генеральный директор ООО «СП «КВАНТ» ______________ Н.Г. Кулагина «___» _________________20___ г. | УТВЕРЖДАЮ Научный директор ООО «МЦКТ» ______________ А.В. Акимов «___» _________________20___ г. |
| --- | --- |

ЧИСЛЕННАЯ МОДЕЛЬ КВАНТОВОЙ ИНТЕГРАЛЬНОЙ МИКРОСХЕМЫ ИЗ НЕ МЕНЕЕ 16 КУБИТОВ ЛИСТ УТВЕРЖДЕНИЯ Текст программы МЦКТ.002.021 ПО ТП-ЛУ

| СОГЛАСОВАНО | СОГЛАСОВАНО |
| --- | --- |
| Представители ООО «СП «КВАНТ» | Представители ООО «МЦКТ» |
| _______________ «___» _______________20___г. | _______________ Толстобров А.Е. «___» _______________20___г. |
| _______________ «___» _______________20___г. | _______________ Федоров Г.П. «___» _______________20___г. |

20___ г.

Москва

Текст программы

import numpy as np

from qutip import *

from numpy import pi, sqrt, exp

class Transmon: #создание класса для описания кубита-трансмона

def __init__(self, Ec, Ej, d, gamma_rel, gamma_phi,

Nc, N_trunc, index, nonlinear_osc=False):

self._Ec = Ec #зарядовая энергия

self._Ej = Ej #джозефсоновская энергия

self._d = d # параметр асимметрии

self._Nc = Nc # число зарядовых состояний

self._Ns = Nc * 2 + 1

self._gamma_rel = gamma_rel #

self._gamma_phi = gamma_phi #

self.index = index

self._nonlinear_osc = nonlinear_osc

self._hd_amplitude = 0.5

self._drive_shape = 'rect' #

self._drive_sigma = 5

self._linear_osc_H_stub = (sqrt(8 * Ej * Ec) - Ec) * create(N_trunc) * destroy(N_trunc)

self._nonlinear_osc_H_stub = - Ec / 2 * create(N_trunc) * destroy(N_trunc) * \

(create(N_trunc) * destroy(N_trunc) - 1)

self._nonlinear_osc_raising = create(N_trunc)

self._nonlinear_osc_lowering = destroy(N_trunc)

self._nonlinear_osc_n = destroy(N_trunc) + create(N_trunc)

self._N_trunc = N_trunc

self._n_cache = {}

self._c_ops_cache = {}

self._Hdr_RF_RWA_cache = {}

self._Hdr_cont_cache = {}

self._H_diag_trunc_cache = {}

def clear_caches(self):

self._n_cache = {}

self._c_ops_cache = {}

self._Hdr_RF_RWA_cache = {}

self._Hdr_cont_cache = {}

self._H_diag_trunc_cache = {}

def _truncate(self, operator): #представление оператора в обрезанном базисе

return Qobj(operator[:self._N_trunc, :self._N_trunc])

def Hc(self): # Зарядовая часть гамильтониана

return 4 * (self._Ec) * charge(self._Nc) ** 2

def Hj(self, phi): # Туннельная часть гамильтониана

return - self._Ej / 2 * tunneling(self._Ns, 1) * self._phi_coeff(phi)

def H_diag_trunc(self, phi): #представление гамильтониана в измерительном базисе

if self._nonlinear_osc:

return self.H_nonlinear_osc(phi)

try:

return self._H_diag_trunc_cache[phi]

except KeyError:

H_charge_basis = self.Hc() + self.Hj(phi)

#H_fixed_flux = self.Hc() + self.Hj(phi_base)

#evals, evecs = H_fixed_flux.eigenstates()

evals, evecs = H_charge_basis.eigenstates()

H = self._truncate(H_charge_basis.transform(evecs))

self._H_diag_trunc_cache[phi] = H - H[0, 0]

return self._H_diag_trunc_cache[phi]

def H_uu(self, freq): # поправка к гамильтониану при переходе в систему отсчёта накачки

if self._N_trunc == 2:

Huu = freq*2*pi*ket2dm(self.e_state())

elif self._N_trunc == 3:

Huu = freq*2*pi*ket2dm(self.e_state())+2*freq*2*pi*ket2dm(self.f_state())

return Huu

def H_diag_trunc_approx(self, phi): #диагонализация гамильтониана

Ej = self._Ej

Ec = self._Ec

omega_01=sqrt(8*Ec*Ej*self._phi_coeff(phi))

if self._N_trunc == 2:

H = self.e_state()*self.e_state().dag()*(omega_01-Ec)

elif self._N_trunc == 3:

H = self.e_state()*self.e_state().dag()*(omega_01-Ec) + self.f_state()*self.f_state().dag()*(2*omega_01-3*Ec)

return H

def H_td_diag_trunc_approx(self, waveform): # гамильтониан при переменном внешнем поле

Ej = self._Ej

Ec = self._Ec

omega_01=sqrt(8*Ec*Ej*self._phi_coeff(waveform))

H11 = [self.e_state()*self.e_state().dag(),omega_01 - Ec]

if self._N_trunc == 2:

return H11

elif self._N_trunc == 3:

H22 = [self.f_state()*self.f_state().dag(),2*omega_01 - 3*Ec]

return [H11] + [H22]

def _phi_coeff(self, phi): # зависимость джозефсоновской энергии от внешнего поля

return (np.cos(phi * pi)**2 + (self._d)**2 * np.sin(phi * pi) ** 2) ** 0.5

def get_Ns(self):

return self._N_trunc

def get_index(self):

return self.index

def Hdr(self, amplitude, duration, start, phase=0, freq=None, flux = 0): #гамильтониан в присутствии накачки

if freq is None:

freq = self.ge_freq_approx(flux)

if self._nonlinear_osc:

return [self._nonlinear_osc_n,

"%f*cos(2*pi*%.16f*t+%f)*(1+np.sign(t-%f))*(1+np.sign(-t+%f))/4" % \

(amplitude, freq, phase, start, start + duration)]

return [self.n(0) / self.n(0).matrix_element(self.g_state(), self.e_state()),

"%f*cos(2*pi*%.16f*t+%f)*(1+np.sign(t-%f))*(1+np.sign(-t+%f))/4" % \

(amplitude, freq, phase, start, start + duration)]

def Hdr_RF_RWA(self, amplitude, start, duration, phase): # гамильтониан в присутствии импульсной накачки

if self._nonlinear_osc:

op = self._nonlinear_osc_n

else:

op1 = (self.n(0) / self.n(0).matrix_element(self.g_state(),

self.e_state())).data.copy()

for i in range(op1.shape[0]):

for j in range(op1.shape[1]):

if i < j:

op1[i, j] = 0

half = Qobj(op1, dims = [[self._N_trunc],[self._N_trunc]])*np.exp(-1j*phase)

op = half + half.dag()

if self._drive_shape == 'gauss':

sigma = self._drive_sigma

t_middle = start + duration/2

return [op, "0.5*%f*np.exp(-0.5*((t-%f)/%f)**2)*(1+np.sign(t-%f))*(1+np.sign(-t+%f))/4"%\

(amplitude, t_middle, sigma, start, start + duration)]

if self._drive_shape == 'gauss_hdrag':

sigma = self._drive_sigma

t_middle = start + duration/2

half_x = half

half_y = half*exp(-1j*pi/2)

q_x = half_x+half_x.dag()

q_y = half_y+half_y.dag()

return [[q_x, "0.5*%f*(np.exp(-0.5*((t-%f)/%f)**2)-np.exp(-0.5*%f))*(1+np.sign(t-%f))*(1+np.sign(-t+%f))/4"%\

(amplitude, t_middle, sigma, ((t_middle-start)/sigma)**2, start, start + duration)],

[q_y,"%f*0.5*%f*(np.exp(-0.5*((t-%f)/%f)**2)-np.exp(-0.5*%f))*(1+np.sign(t-%f))*\

(1+np.sign(-t+%f))/4*(t-%f)/(%f)**2/%f"%\

(self._hd_amplitude, amplitude, t_middle, sigma, ((t_middle-start)/sigma)**2,

start, start + duration,t_middle,sigma,self._Ec)]]

else:

return [op, "0.5*%f*(1+np.sign(t-%f))*(1+np.sign(-t+%f))/4"%\

(amplitude, start, start + duration)]

#op = self.n(0) / self.n(0).matrix_element(self.g_state(), self.e_state())

#return [op, "0.5*%f*(1+np.sign(t-%f))*(1+np.sign(%f-t))/4"%\

#(amplitude, start, start + duration)]

def inverse_transmon_spectrum(self, nu): #Обращение спектра трансмона

Ej = (2*pi*nu + self._Ec)**2/self._Ec/8

Ej_rel = Ej/self._Ej

cos_phi = (Ej_rel**2 - self._d**2)/(1-self._d**2)

phi = np.arccos(cos_phi)

return phi/np.pi

def n_approx (self, phi): # приближённое представление зарядового оператора в собственном базисе

try:

return self._n_cache_approx[phi]

except:

self._n_cache_approx[phi] = self.n(0)*(self._phi_coeff(phi)**0.25)

return self._n_cache_approx[phi]

def n(self, phi): # точное представление зарядового оператора в собственном базисе

if self._nonlinear_osc:

return self._nonlinear_osc_n

try:

return self._n_cache[phi]

except:

H_charge_basis = self.Hc() + self.Hj(phi)

evals, evecs = H_charge_basis.eigenstates()

self._n_cache[phi] = self._truncate(Qobj(abs(charge(self._Nc).transform(evecs))))

return self._n_cache[phi]

def g_state(self): # основное состояние

#         evals, evecs = self.H(phi).eigenstates()

#         return evecs[0]

return basis(self._N_trunc, 0)

def e_state(self): # первое возбуждённое состояние

return basis(self._N_trunc, 1)

def f_state(self): # вторео возбуждённое состояние

return basis(self._N_trunc, 2)

def sz(self): # проекция спина на ось z

return -ket2dm(basis(self._N_trunc, 0)) + ket2dm(basis(self._N_trunc, 1))

def sx(self):  # проекция спина на ось x

return basis(self._N_trunc, 0) * basis(self._N_trunc, 1).dag() + basis(self._N_trunc, 1) * basis(self._N_trunc, 0).dag()

def sy(self):  # проекция спина на ось y

return 1j * basis(self._N_trunc, 0) * basis(self._N_trunc, 1).dag() - 1j * basis(self._N_trunc, 1) * basis(self._N_trunc, 0).dag()

def raising(self, phi): #повышающий оператор

if self._nonlinear_osc:

return self._nonlinear_osc_raising

evecs = [basis(self._N_trunc, i) for i in range(self._N_trunc)]

return sum([abs(self.n(phi).matrix_element(evecs[j + 1], evecs[j])) /

abs(self.n(phi).matrix_element(evecs[0], evecs[1])) *

evecs[j + 1] * evecs[j].dag() for j in range(0, self._N_trunc - 1)])

def lowering(self, phi): #понижающий оператор

if self._nonlinear_osc:

return self._nonlinear_osc_lowering

evecs = [basis(self._N_trunc, i) for i in range(self._N_trunc)]

return sum([abs(self.n(phi).matrix_element(evecs[j], evecs[j + 1])) /

abs(self.n(phi).matrix_element(evecs[0], evecs[1])) *

evecs[j] * evecs[j + 1].dag() for j in range(0, self._N_trunc - 1)])

def rotating_dephasing(self, phi): #оператор числа частиц

return self.raising(phi) * self.lowering(phi)

def c_ops(self, phi): #операторы коллапса

try:

return self._c_ops_cache[phi]

except KeyError:

self._c_ops_cache[phi] = [sqrt(self._gamma_rel) * self.lowering(phi),

sqrt(self._gamma_phi) * self.rotating_dephasing(phi)]

return self._c_ops_cache[phi]

# файл для записи параметров системы
# подключение необходимых библиотек
import two_transmons.DoubleTransmonSystem
from two_transmons.DoubleTransmonSystem import *
import two_transmons.VacuumRabiSimulation
from two_transmons.VacuumRabiSimulation import *
import single_transmon.Transmon
from single_transmon.Transmon import *
from ReadoutResonator import *
from qutip import *
from multiprocessing import Pool


durations = linspace(40,120,100)

def calc_time (farg):
    params=farg['params']
    phi=farg['phi']

    # параметры кубитов
    Nc = 7
    Ec1 = 0.25 * 2 * np.pi
    Ec2 = 1.03 * Ec1
    Ecc = 0.01 / 1.2 ** 2 * 2 * np.pi  #
    Ej1 = Ec1 * 100
    Ej2 = Ec2 * 100
    d = 0.2
    #
    T1_1, T2_1 = 10e3, 5e3 # времена когерентности и релаксации
    T1_2, T2_2 = 5e2, 1e2 # времена когерентности и реелаксации

    tr1 = Transmon(Ec1, Ej1, d, 1 / T1_1, 1 / T2_1, Nc, 3, 1)
    tr2 = Transmon(Ec2, Ej2, d, 1 / T1_2, 1 / T2_2, Nc, 3, 2)
    dts = DoubleTransmonSystem(tr1, tr2, Ecc)
    r = ReadoutResonator(6, 1118, 1964, phi=0.29)
    r.set_qubit_parameters(.05, 0.05, 5, 5.3, .2, .2)
    ar1 = []
    ar2 = []
    for dur in durations:
        Ts = linspace(params['start'],params["finish"],params['t_points'])      
        params1 = params
        params1['phi_offset'] = phi
        params1['phi_base_level']=0
        params1['duration'] = dur
        VRS = VacuumRabiSimulation(dts, Ts, params1, r)
        result = VRS.run()
        state1 = VRS._rho0
        state2 = VRS._dts.e_state(1 / 2, 1 / 2, 2)
        state2 = state2 * state2.dag()
        
        ar1.append(expect(result.states[-1],state1))
        ar2.append(expect(result.states[-1],state2))
        
    ret = {'projections1': ar1, 'projections2': ar2}
    return ret
# вычисление проекций состояний кубитов на ось z в параллельном режиме
def calc_projections(farg):
    projections1 = []
    projections2 = []
    result=[]

    with Pool(24) as p:
        result = []
        for item in tqdm(p.imap(calc_time, farg)):
            result.append(item)
    #print (result)        
    for ind in result:     
        projections1.append(ind['projections1'])
        projections2.append(ind['projections2'])
    return (projections1, projections2)
# задание параметров системы
def cook_farg(Nphi,freq):
    phis = linspace(0.44, 0.52, Nphi)
    params = {
        'duration': 19, #длительность взаимодействия двух кубитов
        'duration_cphase': 350.8,
        'tanh_sigma': 0.5,
        "start": 10, #время начала эксперимента
        "start_cphase": 200,
        "finish": 400, #время окончания эксперимента
        'phi_base_level':0,
        # Положения рабочих точек каждого из кубитов
        "phi_base_levels": { 0 : 0.0, 1 : 0.5, 2 : 0.5, 3 : 0.0,
                            4 : 0.5, 5 : 0.0, 6 : 0.5, 7 : 0.0, 
                            8 : 0.5, 9 : 0.0, 10 : 0.5, 11 : 0.0,
                            12 : 0.5, 13 : 0.0, 14 : 0.5, 15: 0.0},
        # отстройки по фазе для потоковых импульсов
        'phi_offsets': {'I':0.2, 'II':-0.303, 'III':0.19, 'IV':-0.302,
                        'V':0.195, 'VI':-0.314, 'VII':0.196, 'VIII':-0.307,
                       'IX':0.2, 'X':-0.303, 'XI':0.19, 'XII':-0.302,},
        # длительность двухкубитных взаимодействий
        'vro_durations':{'I-II':19.57,
                         'II-III':18.47,
                         'III-IV':17.9,
                         'IV-V':19.0,
                         'V-VI':18.0,
                         'VI-VII':18.0,
                         'VII-VIII':19.5
                        },
        "t_points": 10001, #размер сетки по времени
        "t_zgate" : 10.04, 
        "t_zgate2": 9.8+0.15,   
        't_zgate_2iswap': 11.60,
        't_zgate2_2iswap': 11.60,
        'gauss_pulse_duration':30, #длительность импульса накачки
        'gauss_pi_pulse_ampl': {'I':0.261, 'II':0.258, 'III':0.258, 'IV':0.258,
                                'V':0.258, 'VI':0.258, 'VII':0.258, 'VIII':0.258,
                                'IX':0.258, 'X':0.258, 'XI':0.258, 'XII':0.258},
        'gauss_pi/2_pulse_ampl': {'I':0.25855/2, 'II':0.2592/2, 'III':None, 'IV':None,
                                   'V':None, 'VI':None, 'VII':None, 'VIII':None},
        #"phiz_offset" : 0.1,
        "phiz_offset" : -0.01,
        "phi2z_base_level": 0.3,
        "phi2z_offset" : 0.03,
        #"phi2z_offset" : -0.05,
        "frequency": freq,
        "phi_offset_cphase":0.453894-0.4479,
        "start_cphase": 200,
        "finish_cphase": 200,
        'drive_amplitude':1/30.0*2*pi/2, #амплитуда накачки
        'pi_pulse_duration':30.0
    }
    farg=[]

    for i in range (len(phis)):
        farg.append({'phi' : phis[i],'params' : params})
    return farg

from scipy import *
from qutip import *
from matplotlib.pyplot import *
from tqdm import *


class DoubleTransmonSystem: # класс, описывающий пару трансмонов

    def __init__(self, tr1, tr2, g):
        self._tr1 = tr1
        self._tr2 = tr2
        self._tr_list = [tr1, tr2]
        self._g = g

        self._Huu_cache = {}
        
    def clear_caches(self):
        self._Huu_cache = {}
        self._tr1.clear_caches()
        self._tr2.clear_caches()

    def two_qubit_operator(self, qubit1_operator=None, qubit2_operator=None): #тензорное произведение двух однокубитных операторов

        mask = [identity(self._tr1.get_Ns()), identity(self._tr2.get_Ns())]

        if qubit1_operator is not None:
            mask[0] = qubit1_operator
        if qubit2_operator is not None:
            mask[1] = qubit2_operator

        return tensor(*mask)

    def H(self, phi1, phi2): #Общий гамильтониан системы
        H = self.two_qubit_operator(qubit1_operator=self._tr1.H_diag_trunc(phi1)) + \
            self.two_qubit_operator(qubit2_operator=self._tr2.H_diag_trunc(phi2)) + \
            self.Hint(phi1, phi2)
        return H - H[0, 0]

    def H_RF_RWA(self, phi1, phi2, rotating_frame_freq): #Гамильтониан в приближении вращающейся волны
        H = self.two_qubit_operator(qubit1_operator=self._tr1.H_diag_trunc(phi1)) + \
            self.two_qubit_operator(qubit2_operator=self._tr2.H_diag_trunc(phi2)) + \
            self.Hint_RF_RWA(phi1, phi2)

        return H - H[0, 0] - self._Huu(rotating_frame_freq)
    
    
    def H_iswap_RF_RWA_td(self, waveform1, waveform2, params, rotations): #Зависимость гамильтониана от времени
        ## H1
        dur1, dur2 = rotations[0][0], rotations[1][0]
        phi1, phi2 = rotations[0][1], rotations[1][1]
        flux1 = params['phi_base_level']
        Htd1 = self._tr1.H_td_diag_trunc_approx(waveform1)
        evals1, evecs1 = self._tr1.H_diag_trunc_approx(flux1).eigenstates()
        freq1 = (evals1[1]- evals1[0])/2/pi
        Hdr1 = self._tr1.Hdr_RF_RWA(params['drive_amplitude'], params['start'] - dur1 - 10, dur1, phi1)
        Huu1 = self._tr1.H_uu(freq1)*(-1)
        H1 = Htd1 + [Hdr1] + [Huu1]
        H1_2q = [[self.two_qubit_operator(qubit1_operator = H1[0][0]),H1[0][1]],
        [self.two_qubit_operator(qubit1_operator = H1[1][0]),H1[1][1]],
        [self.two_qubit_operator(qubit1_operator = H1[2][0]),H1[2][1]],
        self.two_qubit_operator(qubit1_operator = H1[3])]
        ## H2
        flux2 = params['phi2z_base_level']
        Htd2 = self._tr2.H_td_diag_trunc_approx(waveform2)
        evals2, evecs2 = self._tr2.H_diag_trunc_approx(flux2).eigenstates()
        freq2 = (evals2[1]- evals2[0])/2/pi
        Hdr2 = self._tr2.Hdr_RF_RWA(params['drive_amplitude'], params['start'] - dur2 - 10, dur2, phi2)
        Huu2 = self._tr2.H_uu(freq2)*(-1)
        H2 = Htd2 + [Hdr2] + [Huu2]
        H2_2q = [[self.two_qubit_operator(qubit2_operator = H2[0][0]),H2[0][1]],
        [self.two_qubit_operator(qubit2_operator = H2[1][0]),H2[1][1]],
        [self.two_qubit_operator(qubit2_operator = H2[2][0]),H2[2][1]],
        self.two_qubit_operator(qubit2_operator = H2[3])]
        Hint = self.Hint_RF_RWA(flux1, flux2, (freq2-freq1)*2*pi, td = True)
        H = H1_2q + H2_2q + Hint
        #print (dur1, dur2, phi1 ,phi2)
        return H
    

    def _Huu(self, freq): #Часть, связанная с переходом во вращающуюся систему отсчёта
        try:
            return self._Huu_cache[freq]
        except:
            self._Huu_cache[freq] = freq * 2 * pi * self.two_qubit_operator(
                qubit1_operator=ket2dm(self._tr1.e_state())) + \
                                    2 * freq * 2 * pi * self.two_qubit_operator(
                qubit1_operator=ket2dm(self._tr1.f_state())) + \
                                    freq * 2 * pi * self.two_qubit_operator(
                qubit2_operator=ket2dm(self._tr2.e_state())) + \
                                    2 * freq * 2 * pi * self.two_qubit_operator(
                qubit2_operator=ket2dm(self._tr2.f_state()))
            return self._Huu_cache[freq]

    def H_td(self, waveform1, waveform2):
        return [self.Hc()] + self.Hj_td(waveform1, waveform2) + [self.Hint()]
    
    def H_td_diag_approx_str(self, params, iswap2 = True): #Часть гамильтониана, зависящая от времени
        waveform1_iswap1 = 't-t'
        waveform1_iswap2 = 't-t '
        waveform1_zgate1 = 't-t '
        waveform1_zgate2 = 't-t '
        waveform2_zgate2 = 't-t '
        if (iswap2):
            gap = 120.00
            waveform1_iswap2 = self._tr1.waveform_td(params['phi_offset'],params['start'] + gap,params['start'] + params['duration'] +gap)
            waveform1_zgate2 = self._tr1.waveform_td(params['phiz_offset'], params['start'] + 90 +120, 
                                                     params['start'] + 90 + gap + params['t_zgate'])
            waveform2_zgate2 = self._tr2.waveform_td(params['phi2z_offset'], params['start'] + 90 +gap, 
                                                 params['start'] + 90 + gap + params['t_zgate2'])
            
        waveform1_iswap1 = self._tr1.waveform_td(params['phi_offset'],params['start'],params['start'] + params['duration'])
        
        waveform1_iswap =  '( ' + waveform1_iswap1 + ' + ' +  waveform1_iswap2 + ' )'
        #waveform1_zgate = 't-t '
        waveform1_zgate1 = self._tr1.waveform_td(params['phiz_offset'], params['start'] + 90, params['start'] + 90 + params['t_zgate'])
        
        waveform1_zgate = '( ' + waveform1_zgate1 + ' + ' +  waveform1_zgate2 + ' )'
        waveform1_sum = '( ' + waveform1_iswap + ' + ' + waveform1_zgate + ' + ' + ' %f'%params['phi_base_level']  + ' )'      
        #waveform2_zgate = 't-t '
        waveform2_zgate1 = self._tr2.waveform_td(params['phi2z_offset'], params['start'] + 90, params['start'] + 90 + params['t_zgate2'])
        
        waveform2_zgate = '( ' + waveform2_zgate1 + ' + ' +  waveform2_zgate2 + ' )'
        
        waveform2_sum = '( ' + waveform2_zgate + ' + ' + ' %f'%params['phi2z_base_level'] + ' )'
        Hj_td1 = self._tr1.H_td_diag_trunc_approx_str(waveform1_sum)        
        Hj_td1[0] = self.two_qubit_operator(qubit1_operator=Hj_td1[0])
       
        Hj_td2 = self._tr2.H_td_diag_trunc_approx_str(waveform2_sum)
        Hj_td2[0] = self.two_qubit_operator(qubit2_operator=Hj_td2[0])
       
        Hint = [self.Hint_RF_RWA( 0.4770,0.5),"1"]
        return [Hj_td1, Hj_td2, Hint]
    
    def H_td_diag_approx(self, waveform1, waveform2): #Приближённое значение гамильтониана
        Hj_td1 = self._tr1.H_td_diag_trunc_approx(waveform1)
        Hj_td1[0] = self.two_qubit_operator(qubit1_operator=Hj_td1[0])
        Hj_td2 = self._tr2.H_td_diag_trunc_approx(waveform2)
        Hj_td2[0] = self.two_qubit_operator(qubit2_operator=Hj_td2[0])
        #return [Hj_td1, Hj_td2, self.Hint(waveform1[0], waveform2[0])]
        return [Hj_td1, Hj_td2, self.Hint_RF_RWA_waveform(waveform1, waveform2)]
        #return [self.Hint_RF_RWA_waveform(waveform1, waveform2)]

    def H_diag_approx(self, phi1, phi2):
        return self.two_qubit_operator(qubit1_operator=self._tr1.H_diag_trunc_approx(phi1)) + \
               self.two_qubit_operator(qubit2_operator=self._tr2.H_diag_trunc_approx(phi2)) + \
               self.Hint_RF_RWA(0.4770, 0.5)

    def Hc(self): #Зарядовая часть гамильтониана
        return self.two_qubit_operator(qubit1_operator=self._tr1.Hc()) + \
               self.two_qubit_operator(qubit2_operator=self._tr2.Hc())

    def Hj(self, phi1, phi2): #Туннельная часть гамильтониана
        return self.two_qubit_operator(qubit1_operator=self._tr1.Hj(phi1)) + \
               self.two_qubit_operator(qubit2_operator=self._tr2.Hj(phi2))

    def Hj_td(self, waveform1, waveform2): #Зависимость туннельной части гамильтониана от времени
        Hj_td1 = self._tr1.Hj_td(waveform1)
        Hj_td1[0] = self.two_qubit_operator(qubit1_operator=Hj_td1[0])
        Hj_td2 = self._tr2.Hj_td(waveform2)
        Hj_td2[0] = self.two_qubit_operator(qubit2_operator=Hj_td2[0])
        return [Hj_td1, Hj_td2]

    def Hint(self, phi1, phi2): #гамильтониан взаимодействия
        return self.two_qubit_operator(self._tr1.n(phi1), self._tr2.n(phi2)) * self._g

    def Hint_RF_RWA(self, phi1, phi2, offset = 0.02, td = False): #гамильтониан взаимодействия в приближении вращающейся волны
        op1 = self._tr1.n(phi1).data.copy()
        op2 = self._tr2.n(phi2).data.copy()
        # zeroing triangles
        for i in range(op1.shape[0]):
            for j in range(op1.shape[1]):
                if i < j:
                    op1[i, j] = 0
                if i > j:
                    op2[i, j] = 0
        half = self.two_qubit_operator(Qobj(op1), Qobj(op2))
        if td:
            return [[self._g * half, "exp(-1j*(%f)*t)"%offset],[self._g * half.dag(), "exp(1j*(%f)*t)"%offset]]
        else:
            return self._g * (half + half.dag())
    def Hint_RF_RWA_waveform(self, waveform1, waveform2, td = False):
        return [self.Hint_RF_RWA(0, 0.5), array([self._tr1._phi_coeff(phi)**0.25 for phi in waveform1])]
                #array([(self._tr1._phi_coeff(waveform1[i])*self._tr2._phi_coeff(waveform2[i])) for i in range(len (waveform1))])]

    def Hdr(self, amplitudes, durations, starts, phases, freqs,fluxes): #Гамильтониан в присутствии импульсной накачки
        #Hdr1 = self._tr1.Hdr_cont(amplitudes[0], durations[0], starts[0], phases[0])
        Hdr1 = self._tr1.Hdr(amplitudes[0], durations[0], starts[0], phases[0], freqs[0], fluxes[0])
        Hdr1 = [self.two_qubit_operator(qubit1_operator=Hdr1[0]), Hdr1[1]]

        #Hdr2 = self._tr2.Hdr(amplitudes[1], durations[1], starts[1], phases[1])
        Hdr2 = self._tr2.Hdr(amplitudes[1], durations[1], starts[1], phases[1], freqs[1], fluxes[1])
        Hdr2 = [self.two_qubit_operator(qubit2_operator=Hdr2[0]), Hdr2[1]]

        return [Hdr1] + [Hdr2]

    def Hdr_cont(self, amplitudes): #гамильтониан в присутствии непрерывной накачки
        Hdr1 = self._tr1.Hdr_cont(amplitudes[0])
        Hdr1 = [self.two_qubit_operator(qubit1_operator=Hdr1[0]), Hdr1[1]]

        Hdr2 = self._tr2.Hdr_cont(amplitudes[1])
        Hdr2 = [self.two_qubit_operator(qubit2_operator=Hdr2[0]), Hdr2[1]]

        return [Hdr1] + [Hdr2]

    def Hdr_cont_RF_RWA(self, amplitudes): #гамильтониан в присутствии непрерывной накачки в приближении вращающейся волны
        return [self.two_qubit_operator(qubit1_operator=self._tr1.Hdr_cont_RF_RWA(amplitudes[0]))]+\
        [self.two_qubit_operator(qubit2_operator=self._tr2.Hdr_cont_RF_RWA(amplitudes[1]))]

    def _remove_global_phase(self, state): #удаление глобальной фазы
        state_full = state.full()
        return state * sign(state_full[argmax(abs(state_full))])[0]

    def gg_state(self, phi1, phi2, energy=False): # основное состояние
        evals, evecs = self.H_diag_approx(phi1, phi2).eigenstates()
        evec = self._remove_global_phase(evecs[0])

        return evec if not energy else (evec, evals[0])

    def e_state(self, phi1, phi2, qubit_idx, energy=False): # возбуждённое состояние
        evals, evecs = self.H_diag_approx(phi1, phi2).eigenstates()

        if qubit_idx == 1:
            model_state = self.two_qubit_operator(self._tr1.e_state(), self._tr2.g_state())
        elif qubit_idx == 2:
            model_state = self.two_qubit_operator(self._tr1.g_state(), self._tr2.e_state())

        for idx, evec in enumerate(evecs):
            if abs((evec.dag() * model_state).full()) > 0.6:
                evec = self._remove_global_phase(evec)
                return evec if not energy else (evec, evals[idx])
    
    def f_state(self, phi1, phi2, qubit_idx, energy=False): # второе возбуждённое состояние
        evals, evecs = self.H_diag_approx(phi1, phi2).eigenstates()

        if qubit_idx == 1:
            model_state = self.two_qubit_operator(self._tr1.f_state(), self._tr2.g_state())
        elif qubit_idx == 2:
            model_state = self.two_qubit_operator(self._tr1.g_state(), self._tr2.f_state())

        for idx, evec in enumerate(evecs):
            if abs((evec.dag() * model_state).full()) > 0.6:
                evec = self._remove_global_phase(evec)
                return evec if not energy else (evec, evals[idx])

    def ee_state(self, phi1, phi2, energy=False): # возбуждение 2 кубитов одновременно
        evals, evecs = self.H_diag_approx(phi1, phi2).eigenstates()
        model_state = self.two_qubit_operator(self._tr1.e_state(), self._tr2.e_state())
        for idx, evec in enumerate(evecs):
            if abs((evec.dag() * model_state).full()) > 0.6:
                evec = self._remove_global_phase(evec)
                return evec if not energy else (evec, evals[idx])

    def c_ops(self, phi1, phi2): # операторы коллапса
        c_ops = []
        c_ops1 = self._tr1.c_ops(phi1)
        for c_op in c_ops1:
            c_ops.append(self.two_qubit_operator(qubit1_operator=c_op))
        c_ops2 = self._tr2.c_ops(phi2)
        for c_op in c_ops2:
            c_ops.append(self.two_qubit_operator(qubit2_operator=c_op))
        return c_ops

    def plot_spectrum(self, phi1s, phi2s, exact_H = True, currents=None): # отрисовка спектра системы
        assert len(phi1s) == len(phi2s)

        fluxes = list(zip(phi1s, phi2s))

        fixed_flux_spectra = []
        if (exact_H):
            for phi1, phi2 in tqdm_notebook(fluxes, desc='Energy levels calculation'):
                H = self.H(phi1, phi2)
                evals = H.eigenenergies()
                fixed_flux_spectra.append(evals)
        else:
            for phi1, phi2 in tqdm_notebook(fluxes, desc='Energy levels calculation'):
                H_approx = self.H_diag_approx(phi1, phi2)
                evals = H_approx.eigenenergies()
                fixed_flux_spectra.append(evals)

        fixed_flux_spectra = array(fixed_flux_spectra)
        eigenlevels = fixed_flux_spectra.T
        transitions_from_g = eigenlevels - eigenlevels[0]
        
        self._single_photon_transitions = transitions_from_g[1:3].T / 2 / pi
        self._two_photon_transitions = transitions_from_g[3:6].T / 2 / pi / 2

        if currents is not None:
            plot(currents, transitions_from_g[1:3].T / 2 / pi)
            plot(currents, transitions_from_g[3:6].T / 2 / pi / 2)
        else:
            plot(phi1s, transitions_from_g[1:3].T / 2 / pi)
            plot(phi1s, transitions_from_g[3:6].T / 2 / pi / 2)

        grid()

    
    def plot_per_qubit_xyz_dynamics(self, Ts, result): #отрисовка динамики системы
        X1, Y1, Z1 = [], [], []
        X2, Y2, Z2 = [], [], []
        X1 = result.expect[0]
        Y1 = result.expect[1]
        Z1 = result.expect[2]
        X2 = result.expect[3]
        Y2 = result.expect[4]
        Z2 = result.expect[5]
        fig, axes = subplots(2, 1, figsize=(15, 6))
        axes[0].plot(Ts, X1, label=r"$\langle x\rangle$")
        axes[0].plot(Ts, Y1, label=r"$\langle y\rangle$")
        axes[0].plot(Ts, Z1, label=r"$\langle z\rangle$")
        axes[1].plot(Ts, X2, label=r"$\langle x\rangle$")
        axes[1].plot(Ts, Y2, label=r"$\langle y\rangle$")
        axes[1].plot(Ts, Z2, label=r"$\langle z\rangle$")
        for ax in axes:
            ax.grid()
            ax.legend()
        return fig, axes

import two_transmons.DoubleTransmonSystem
from two_transmons.DoubleTransmonSystem import *
import two_transmons.VacuumRabiSimulation
from two_transmons.VacuumRabiSimulation import *
import single_transmon.Transmon
from single_transmon.Transmon import *
from ReadoutResonator import *
from qutip import *


def calculate (phi):
    Nc = 7
    Ec1 = 0.25 * 2 * np.pi
    Ec2 = 1.03 * Ec1
    Ecc = 0.01 / 1.2 ** 2 * 2 * np.pi  # 1.2 is my estimate for n_{ge} matrix element
    Ej1 = Ec1 * 100
    Ej2 = Ec2 * 100
    d = 0.2

    T1_1, T2_1 = 10e3, 5e3
    T1_2, T2_2 = 5e2, 1e2

    tr1 = Transmon(Ec1, Ej1, d, 1 / T1_1, 1 / T2_1, Nc, 3, 1)
    tr2 = Transmon(Ec2, Ej2, d, 1 / T1_2, 1 / T2_2, Nc, 3, 2)
    dts = DoubleTransmonSystem(tr1, tr2, Ecc)
    r = ReadoutResonator(6, 1118, 1964, phi=0.29)
    r.set_qubit_parameters(.05, 0.05, 5, 5.3, .2, .2)
    Ts = linspace(0, params["finish"], params['t_points']) 
    params1=params
    params1['phi_offset']=phi
    VRS = VacuumRabiSimulation(dts, Ts, params1, r)
    result = VRS.run()

    state1 = VRS._rho0
    state2 = VRS._dts.e_state(1 / 2, 1 / 2, 2)
    state2 = state2 * state2.dag()
    ar1 = []
    ar2 = []

    for state in result.states:
        if len(VRS._c_ops) == 0:
            state = state * state.dag()
        ar1.append(expect(state, state1))
        ar2.append(expect(state, state2))
    ret = {'projections1': ar1, 'projections2': ar2}
    return ret
def f1 (x):
    ret= {'r1' : x*x,'r2': x}
    return ret

#подключение необходимых библиотек
from two_transmons.ZPulse import *
from numpy import ones_like
def vacuum_rabi_waveform_I_II (duration, phi_offset, Ts, params):
    signal = ZPulse(Ts,params)
    waveforms = []   
    waveform1 = signal._normalized_pulse(params['start'], params['start'] + duration)*params['phi_offsets']['I'] +\
    params['phi_base_levels']['I']
    waveform2 = ones_like(Ts)*(params['phi_base_levels']['II']) +\
        signal._normalized_pulse(params['start'], params['start'] + duration)*phi_offset
    waveforms.extend([waveform1,waveform2])
    for qubit in params['phi_base_levels'].keys():
        if qubit not in ['I','II'] :
            waveforms.append(ones_like(Ts)*(params['phi_base_levels'][qubit]))   
    return waveforms

    

def vacuum_rabi_waveform_I_III (duration, phi_offset, Ts, params):
    '''
    Данная функция описывает распространение возбуждения от 1го кубита до 3го используя 2 двухкубитные операции
    '''
    signal = ZPulse(Ts,params)
    waveforms=[]
    t_begin = params['start'] + params['vro_durations']['I-II'] + 10
    waveform1 = signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['I']\
    +params['phi_base_levels']['I']
    waveform2 = ones_like(Ts)*(params['phi_base_levels']['II']) +\
    signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['II'] +\
    signal._normalized_pulse(t_begin, t_begin + duration)*params['phi_offsets']['II']
    waveform3 = ones_like(Ts)*(params['phi_base_levels']['III'])+\
    signal._normalized_pulse(t_begin, t_begin + duration)*phi_offset
    waveforms.extend([waveform1,waveform2,waveform3])
    for qubit in params['phi_base_levels'].keys():
        if qubit not in ['I','II','III'] :
            waveforms.append(ones_like(Ts)*(params['phi_base_levels'][qubit]))   
    return waveforms


def vacuum_rabi_waveform_I_IV (duration, phi_offset, Ts, params):
    signal = ZPulse(Ts,params)
    waveforms=[]
    t_begin_II_III = params['start'] + params['vro_durations']['I-II'] + 10
    t_begin_III_IV = t_begin_II_III + params['vro_durations']['II-III'] + 10
    waveform1 = signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['I']\
    +params['phi_base_levels']['I']
    waveform2 = ones_like(Ts)*(params['phi_base_levels']['II']) +\
    signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['II'] +\
    signal._normalized_pulse(t_begin_II_III, t_begin_II_III + params['vro_durations']['II-III'])*params['phi_offsets']['II']
    waveform3 = ones_like(Ts)*(params['phi_base_levels']['III'])+\
    signal._normalized_pulse(t_begin_II_III, t_begin_II_III + params['vro_durations']['II-III'])*params['phi_offsets']['III']+\
    signal._normalized_pulse(t_begin_III_IV, t_begin_III_IV + duration)*params['phi_offsets']['III']
    waveform4 = ones_like(Ts)*(params['phi_base_levels']['IV'])+\
    signal._normalized_pulse(t_begin_III_IV, t_begin_III_IV + duration)*phi_offset
    
    
    waveforms.extend([waveform1,waveform2,waveform3, waveform4])
    for qubit in params['phi_base_levels'].keys():
        if qubit not in ['I','II','III', 'IV'] :
            waveforms.append(ones_like(Ts)*(params['phi_base_levels'][qubit]))   
    return waveforms


def vacuum_rabi_waveform_I_V (duration, phi_offset, Ts, params):
    signal = ZPulse(Ts,params)
    waveforms=[]
    t_begin_II_III = params['start'] + params['vro_durations']['I-II'] + 10
    t_begin_III_IV = t_begin_II_III + params['vro_durations']['II-III'] + 10
    t_begin_IV_V = t_begin_III_IV + params['vro_durations']['III-IV'] + 10
    waveform1 = signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['I']\
    +params['phi_base_levels']['I']
    waveform2 = ones_like(Ts)*(params['phi_base_levels']['II']) +\
    signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['II'] +\
    signal._normalized_pulse(t_begin_II_III, t_begin_II_III + params['vro_durations']['II-III'])*params['phi_offsets']['II']
    waveform3 = ones_like(Ts)*(params['phi_base_levels']['III'])+\
    signal._normalized_pulse(t_begin_II_III, t_begin_II_III + params['vro_durations']['II-III'])*params['phi_offsets']['III']+\
    signal._normalized_pulse(t_begin_III_IV, t_begin_III_IV + params['vro_durations']['III-IV'])*params['phi_offsets']['III']
    waveform4 = ones_like(Ts)*(params['phi_base_levels']['IV'])+\
    signal._normalized_pulse(t_begin_III_IV, t_begin_III_IV + params['vro_durations']['III-IV'])*params['phi_offsets']['IV']+\
    signal._normalized_pulse(t_begin_IV_V, t_begin_IV_V + duration)*params['phi_offsets']['IV']
    waveform5 = ones_like(Ts)*(params['phi_base_levels']['V'])+\
    signal._normalized_pulse(t_begin_IV_V, t_begin_IV_V + duration)*phi_offset
    
    
    waveforms.extend([waveform1,waveform2,waveform3, waveform4,waveform5])
    for qubit in params['phi_base_levels'].keys():
        if qubit not in ['I','II','III', 'IV','V'] :
            waveforms.append(ones_like(Ts)*(params['phi_base_levels'][qubit]))   
    return waveforms


def vacuum_rabi_waveform_I_VI (duration, phi_offset, Ts, params):
    signal = ZPulse(Ts,params)
    waveforms=[]
    
    t_begin_II_III = params['start'] + params['vro_durations']['I-II'] + 10
    t_begin_III_IV = t_begin_II_III + params['vro_durations']['II-III'] + 10
    t_begin_IV_V = t_begin_III_IV + params['vro_durations']['III-IV'] + 10
    t_begin_V_VI = t_begin_IV_V + params['vro_durations']['IV-V'] + 10
    
    waveform1 = signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['I']\
    +params['phi_base_levels']['I']
    
    waveform2 = ones_like(Ts)*(params['phi_base_levels']['II']) +\
    signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['II'] +\
    signal._normalized_pulse(t_begin_II_III, t_begin_II_III + params['vro_durations']['II-III'])*params['phi_offsets']['II']
    
    waveform3 = ones_like(Ts)*(params['phi_base_levels']['III'])+\
    signal._normalized_pulse(t_begin_II_III, t_begin_II_III + params['vro_durations']['II-III'])*params['phi_offsets']['III']+\
    signal._normalized_pulse(t_begin_III_IV, t_begin_III_IV +  params['vro_durations']['III-IV'])*params['phi_offsets']['III']
    
    waveform4 = ones_like(Ts)*(params['phi_base_levels']['IV'])+\
    signal._normalized_pulse(t_begin_III_IV, t_begin_III_IV + params['vro_durations']['III-IV'])*params['phi_offsets']['IV']+\
    signal._normalized_pulse(t_begin_IV_V, t_begin_IV_V +  params['vro_durations']['IV-V'])*params['phi_offsets']['IV']
    
    waveform5 = ones_like(Ts)*(params['phi_base_levels']['V'])+\
    signal._normalized_pulse(t_begin_IV_V, t_begin_IV_V + params['vro_durations']['IV-V'])*params['phi_offsets']['V']+\
    signal._normalized_pulse(t_begin_V_VI, t_begin_V_VI + duration)*params['phi_offsets']['V']  
    
    waveform6 = ones_like(Ts)*(params['phi_base_levels']['VI'])+\
    signal._normalized_pulse(t_begin_V_VI, t_begin_V_VI + duration)*phi_offset 
    
    
    waveforms.extend([waveform1,waveform2,waveform3, waveform4,waveform5,waveform6])
    for qubit in params['phi_base_levels'].keys():
        if qubit not in ['I','II','III', 'IV','V','VI'] :
            waveforms.append(ones_like(Ts)*(params['phi_base_levels'][qubit]))   
    return waveforms  


def vacuum_rabi_waveform_I_VII (duration, phi_offset, Ts, params):
    signal = ZPulse(Ts,params)
    waveforms=[]
    
    t_begin_II_III = params['start'] + params['vro_durations']['I-II'] + 10
    t_begin_III_IV = t_begin_II_III + params['vro_durations']['II-III'] + 10
    t_begin_IV_V = t_begin_III_IV + params['vro_durations']['III-IV'] + 10
    t_begin_V_VI = t_begin_IV_V + params['vro_durations']['IV-V'] + 10
    t_begin_VI_VII = t_begin_V_VI + params['vro_durations']['V-VI'] + 10
    
    waveform1 = signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['I']\
    +params['phi_base_levels']['I']
    
    waveform2 = ones_like(Ts)*(params['phi_base_levels']['II']) +\
    signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['II'] +\
    signal._normalized_pulse(t_begin_II_III, t_begin_II_III + params['vro_durations']['II-III'])*params['phi_offsets']['II']
    
    waveform3 = ones_like(Ts)*(params['phi_base_levels']['III'])+\
    signal._normalized_pulse(t_begin_II_III, t_begin_II_III + params['vro_durations']['II-III'])*params['phi_offsets']['III']+\
    signal._normalized_pulse(t_begin_III_IV, t_begin_III_IV + params['vro_durations']['III-IV'])*params['phi_offsets']['III']
    
    waveform4 = ones_like(Ts)*(params['phi_base_levels']['IV'])+\
    signal._normalized_pulse(t_begin_III_IV, t_begin_III_IV + params['vro_durations']['III-IV'])*params['phi_offsets']['IV']+\
    signal._normalized_pulse(t_begin_IV_V, t_begin_IV_V + params['vro_durations']['IV-V'])*params['phi_offsets']['IV']
    
    waveform5 = ones_like(Ts)*(params['phi_base_levels']['V'])+\
    signal._normalized_pulse(t_begin_IV_V, t_begin_IV_V + params['vro_durations']['IV-V'])*params['phi_offsets']['V']+\
    signal._normalized_pulse(t_begin_V_VI, t_begin_V_VI + params['vro_durations']['V-VI'])*params['phi_offsets']['V']  
    
    waveform6 = ones_like(Ts)*(params['phi_base_levels']['VI'])+\
    signal._normalized_pulse(t_begin_V_VI, t_begin_V_VI + params['vro_durations']['V-VI'])*params['phi_offsets']['VI']+\
    signal._normalized_pulse(t_begin_VI_VII, t_begin_VI_VII + duration)*params['phi_offsets']['VI']  
    
    waveform7 = ones_like(Ts)*(params['phi_base_levels']['VII'])+\
    signal._normalized_pulse(t_begin_VI_VII, t_begin_VI_VII + duration)*phi_offset 
    
    
    waveforms.extend([waveform1,waveform2,waveform3, waveform4,waveform5,waveform6,waveform7])
    for qubit in params['phi_base_levels'].keys():
        if qubit not in ['I','II','III', 'IV','V','VI','VII'] :
            waveforms.append(ones_like(Ts)*(params['phi_base_levels'][qubit]))   
    return waveforms 

def vacuum_rabi_waveform_I_VIII (duration, phi_offset, Ts, params):
    signal = ZPulse(Ts,params)
    waveforms=[]
    
    t_begin_II_III = params['start'] + params['vro_durations']['I-II'] + 10
    t_begin_III_IV = t_begin_II_III + params['vro_durations']['II-III'] + 10
    t_begin_IV_V = t_begin_III_IV + params['vro_durations']['III-IV'] + 10
    t_begin_V_VI = t_begin_IV_V + params['vro_durations']['IV-V'] + 10
    t_begin_VI_VII = t_begin_V_VI + params['vro_durations']['V-VI'] + 10
    t_begin_VII_VIII = t_begin_VI_VII + params['vro_durations']['VI-VII'] + 10
    
    waveform1 = signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['I']\
    +params['phi_base_levels']['I']
    
    waveform2 = ones_like(Ts)*(params['phi_base_levels']['II']) +\
    signal._normalized_pulse(params['start'], params['start'] + params['vro_durations']['I-II'])*params['phi_offsets']['II'] +\
    signal._normalized_pulse(t_begin_II_III, t_begin_II_III + params['vro_durations']['II-III'])*params['phi_offsets']['II']
    
    waveform3 = ones_like(Ts)*(params['phi_base_levels']['III'])+\
    signal._normalized_pulse(t_begin_II_III, t_begin_II_III + params['vro_durations']['II-III'])*params['phi_offsets']['III']+\
    signal._normalized_pulse(t_begin_III_IV, t_begin_III_IV + params['vro_durations']['III-IV'])*params['phi_offsets']['III']
    
    waveform4 = ones_like(Ts)*(params['phi_base_levels']['IV'])+\
    signal._normalized_pulse(t_begin_III_IV, t_begin_III_IV + params['vro_durations']['III-IV'])*params['phi_offsets']['IV']+\
    signal._normalized_pulse(t_begin_IV_V, t_begin_IV_V + params['vro_durations']['IV-V'])*params['phi_offsets']['IV']
    
    waveform5 = ones_like(Ts)*(params['phi_base_levels']['V'])+\
    signal._normalized_pulse(t_begin_IV_V, t_begin_IV_V + params['vro_durations']['IV-V'])*params['phi_offsets']['V']+\
    signal._normalized_pulse(t_begin_V_VI, t_begin_V_VI + params['vro_durations']['V-VI'])*params['phi_offsets']['V']  
    
    waveform6 = ones_like(Ts)*(params['phi_base_levels']['VI'])+\
    signal._normalized_pulse(t_begin_V_VI, t_begin_V_VI + params['vro_durations']['V-VI'])*params['phi_offsets']['VI']+\
    signal._normalized_pulse(t_begin_VI_VII, t_begin_VI_VII + params['vro_durations']['VI-VII'])*params['phi_offsets']['VI']  
    
    waveform7 = ones_like(Ts)*(params['phi_base_levels']['VII'])+\
    signal._normalized_pulse(t_begin_VI_VII, t_begin_VI_VII + params['vro_durations']['VI-VII'])*params['phi_offsets']['VII']+\
    signal._normalized_pulse(t_begin_VII_VIII, t_begin_VII_VIII + duration)*params['phi_offsets']['VII'] 
    
    waveform8 = ones_like(Ts)*(params['phi_base_levels']['VIII'])+\
    signal._normalized_pulse(t_begin_VII_VIII, t_begin_VII_VIII + duration)*phi_offset 
    
    
    waveforms.extend([waveform1,waveform2,waveform3, waveform4,waveform5,waveform6,waveform7,waveform8])
    for qubit in params['phi_base_levels'].keys():
        if qubit not in ['I','II','III', 'IV','V','VI','VII','VIII'] :
            waveforms.append(ones_like(Ts)*(params['phi_base_levels'][qubit]))   
    return waveforms 

    
# расчёт заселённости первого возбуждённого состояния каждого из кубитов 
def vacuum_rabi_population(chain, params, duration, phi_offset, experiment_type):
    if experiment_type == "I-II":
        waveforms = vacuum_rabi_waveform_I_II(duration, phi_offset, chain._Ts, params)
    elif experiment_type == "I-III":
        waveforms = vacuum_rabi_waveform_I_III(duration, phi_offset, chain._Ts, params)
    elif experiment_type == "I-IV":
        waveforms = vacuum_rabi_waveform_I_IV(duration, phi_offset, chain._Ts, params)
    elif experiment_type == "I-V":
        waveforms = vacuum_rabi_waveform_I_V(duration, phi_offset, chain._Ts, params)
    elif experiment_type == "I-VI":
        waveforms = vacuum_rabi_waveform_I_VI(duration, phi_offset, chain._Ts, params)
    elif experiment_type == "I-VII":
        waveforms = vacuum_rabi_waveform_I_VII(duration, phi_offset, chain._Ts, params)
    elif experiment_type == "I-VIII":
        waveforms = vacuum_rabi_waveform_I_VIII(duration, phi_offset, chain._Ts, params)
    
    else: 
        raise NotImplementedError
    drive_amplitudes = []
    for qubit in params['gauss_pi_pulse_ampl'].keys():
        if qubit == 'I':
            drive_amplitudes.append([params['gauss_pi_pulse_ampl'][qubit],0])
        else:
            drive_amplitudes.append([0,0])   
    H_full = chain.build_H_full(waveforms, 
                                 params, drive_amplitudes)
    result = mesolve(H_full, chain.rho0, chain._Ts, c_ops = chain.build_c_ops(), e_ops = chain.e_ops, 
                     progress_bar = None,options=Options(nsteps = 20000, store_states = True, max_step = 1e-1))
  
    if experiment_type == "I-II":
        return result.expect[1][-1]
    elif experiment_type == "I-III":
        return result.expect[2][-1]
    elif experiment_type == "I-IV":
        return result.expect[3][-1]
    elif experiment_type == "I-V":
        return result.expect[4][-1]
    elif experiment_type == "I-VI":
        return result.expect[5][-1]
    elif experiment_type == "I-VII":
        return result.expect[6][-1]
    elif experiment_type == "I-VIII":
        return result.expect[7][-1]

#расчёт заселённости для одного из значений магнитных потоков    
def vacuum_rabi_populations_one_phi(chain, params, durations, phi_offset, experiment_type):
    populations_one_phi = []
    size = len(durations)
    for dur in durations:
        populations_one_phi.append(vacuum_rabi_population(chain, params, dur, phi_offset, experiment_type))
    return populations_one_phi    
def vacuum_rabi_populations_onephi_2qubits(chain, params, durations, phi_offset):
    populations_one_phi = []
    size = len(durations)
    Ts=chain._Ts
    for dur in durations:
        signal = ZPulse(Ts,params)
        waveform1 = signal._normalized_pulse(200, 200 + dur)*phi_offset + params['phi_base_level']
        waveform2_const = ones_like(Ts)*(params['phi2z_base_level'])
        H_full = chain.build_H_full([waveform1, waveform2_const], 
                                 params, [[33.45*5,0],[0,0]])
        result = mesolve(H_full, chain.rho0, chain._Ts, c_ops = chain.build_c_ops(), e_ops = chain.e_ops, 
                     progress_bar = None,options=Options(nsteps = 20000, store_states = True, max_step = 1e-1))
        num_measure = (200 + dur)/params['finish']*len(Ts) 
        populations_one_phi.append(result.expect[1][int(num_measure)])
    return populations_one_phi 

def vacuum_rabi_populations_one_phi_windows(args):
    chain = args ['chain']
    params = args['params']
    durations = args['durations']
    phi_offset = args['offset_amplitude']   
    experiment_type=args['experiment_type']
    populations_one_phi = []
    size = len(durations)
    for dur in durations:
        populations_one_phi.append(vacuum_rabi_population(chain, params, dur, phi_offset, experiment_type))
    return populations_one_phi
# расчёт многокубитных осцилляций
def multiqubit_oscillations(args):
    chain = args['chain']
    params = args['params']
    offset_amplitude = args['offset_amplitude']
    qubit_idx = args['qubit_idx']
    Ts = args['Ts']
    rho0 = args['rho0']
    
    waveforms, drive_amplitudes = [], []
    for qubit in range(12):    
        waveforms.append(ones_like(Ts)*params['phi_base_levels'][qubit])
        drive_amplitudes.append([0,0])  
    
    signal = ZPulse(Ts, params)
    waveforms[qubit_idx] = signal._normalized_pulse(params['start'], params['finish'])\
    *(offset_amplitude) + params['phi_base_levels'][qubit_idx]

    H_full_test = chain.build_H_full(waveforms, 
                                     params, drive_amplitudes )
    # Вызов функции, решающей уравнение Шредингера, для моделирования эволюции системы
    result = mesolve(H_full_test, rho0, Ts, 
                     c_ops = [], e_ops = chain.e_ops,
                     progress_bar=True, options=Options(nsteps = 20000, store_states = True, max_step = 3e-1))
    return result.expect

# подключение необходимых библиотек
from scipy import *
from qutip import *
from matplotlib.pyplot import *

from two_transmons.ZPulse import *
# класс для расчёта двухкубитных осцилляций
class VacuumRabiSimulation:
    
    def __init__(self, dts, Ts, params, readout_resonator):
        self._dts = dts
        self._Ts = Ts
        self._params = params
        self._r = readout_resonator
        
        self._rho0 = dts.e_state(0, 1/2, 1) 
#       self._rho0 = 1/2*(dts.gg_state(0, 1/2)+dts.e_state(0, 1/2, 1)+dts.e_state(0, 1/2, 2)+dts.ee_state(0, 1/2))
        self._rho0 = self._rho0*self._rho0.dag()
#       self._rho0 = tensor(dts._tr1.e_state(params["phi_base_level"]), dts._tr2.g_state(1/2))
    # задание формы управляющего сигнала
    def build_waveforms(self):
        waveform1 = ZPulse(self._Ts, self._params).waveform()
        waveform2 = ones_like(self._Ts)*1/2
        return waveform1, waveform2
        
    def run(self):
        options = Options(nsteps=20000, store_states=True)
        
        self._c_ops = []  #dts.c_ops(1/2, 1/2)
        
        self._result = mesolve(self._dts.H_td_diag_approx(*self.build_waveforms()), self._rho0, 
                               self._Ts, c_ops = self._c_ops, progress_bar=None, options=options)
        return self._result

    # визуализация проекций кубитов
    def visualize_projections(self):
        state1 = self._rho0
        state2 = self._dts.e_state(1/2, 1/2, 2)
        state2 = state2*state2.dag()
        
        projections1 = []
        projections2 = []
        
        for state in self._result.states:
            if len(self._c_ops) == 0:
                state = state*state.dag()
            projections1.append(expect(state, state1))
            projections2.append(expect(state, state2))
        
        plot(self._Ts, np.abs(projections1))
        plot(self._Ts, np.abs(projections2))
        
    def visualize_joint_readout(self, f):
        data = array([expect(state, self._r.measurement_operator(f, self._r.get_dipsersive_shifts())) \
                      for state in self._result.states])
        fig, axes = subplots(1, 2, figsize=(15,5))
        axes[0].plot(self._Ts, data.real)
        axes[1].plot(self._Ts, data.imag, "C1")

# подключение необходимых библиотек
from scipy import *
from qutip import *
from matplotlib.pyplot import *
from numpy import pi, sign, tanh
import math
from scipy.signal import *
from two_transmons.ParallelCalculations import *
# Класс для реализации потокового импульса
class ZPulse:
    
    def __init__(self, Ts, params):
        self._params = params
        self._Ts = Ts
    # импульс с линейным нарастанием
    def _waveform_triangle (self, t_start, t_finish, delta = 0.1):
        tau = delta * (t_finish - t_start)
        wf_rise = (self._Ts - t_start)*(1 + sign(self._Ts-t_start))*(1+ sign(t_start + tau - self._Ts))/4/tau
        wf_down = (t_finish - self._Ts)*(1 + sign(self._Ts-t_finish+tau))*(1 + sign(t_finish - self._Ts))/4/tau
        wf_flat = (1+sign(self._Ts - t_start - tau))*(1+sign(t_finish -tau -self._Ts))/4
        waveform = wf_rise + wf_down + wf_flat
        return waveform
    
    def waveform_triangle_t(self, t, t_start, t_finish, delta = 0.1):
        tau = delta * (t_finish - t_start)
        wf_rise = (t - t_start)*(1 + sign(t-t_start))*(1+ sign(t_start + tau - t))/4/tau
        wf_down = (t_finish - t)*(1 + sign(t-t_finish+tau))*(1 + sign(t_finish - t))/4/tau
        wf_flat = (1+sign(t - t_start - tau))*(1+sign(t_finish -tau -t))/4
        waveform = wf_rise + wf_down + wf_flat
        return waveform 
    def phi_coeffs_t (self, t, t_start, t_finish):
        return self.waveform_triangle_t
        
    # тангенциальное нарастание потока
    def _step_rising(self, t_start = None):
        params = self._params
        if t_start is None:
            t_start = params["start"]        
        return (tanh((self._Ts - 2 * params["tanh_sigma"] - t_start) / params["tanh_sigma"])+1)/2
    # тангенциальное спадание потока
    def _step_falling(self, t_finish = None):
        params = self._params
        if t_finish is None:
            t_finish =  params["start"] + params["duration"]        
        return (tanh((-self._Ts - 2 * params["tanh_sigma"] + t_finish) / params["tanh_sigma"])+1)/2

    #нормализация импульса
    def _normalized_pulse(self, t_start = None, t_finish = None):
        params = self._params
        if t_start is None:
            t_start = params["start"]  
        if t_finish is None:
            t_finish =  params["start"] + params["duration"]
        raw = self._step_rising(t_start)*self._step_falling(t_finish)
        normalized = raw/max(raw)
        return normalized
        #return self._waveform_triangle(t_start,t_finish,0.1)
    def waveform1(self, t_start = None, t_finish = None):
        params = self._params
        if t_start is None:
            t_start = params["start"]  
        if t_finish is None:
            t_finish =  params["start"] + params["duration"]
        offset = self._params["phi_offset"]
        base = self._params["phi_base_level"]
        #return self._waveform_triangle(t_start, t_finish, 0.1)*offset + base
        return self._normalized_pulse(t_start, t_finish)*offset+base
    
    def waveform (self,t_start = None, t_finish = None, fs=1):        
        params = self._params
        if t_start is None:
            t_start = params["start"]  
        if t_finish is None:
            t_finish =  params["start"] + params["duration"]
        #sos=butter(1,self._params['frequency'],btype='lowpass', output='sos', fs=fs)
        #wf1=self.waveform1(t_start, t_finish)
        #filtered=sosfilt(sos,wf1-wf1[0])
        #return filtered+wf1[0]
        return self.waveform1(t_start, t_finish)
    
    # форма импульса для реализации двухкубитной операции
    def waveform_2iswap(self, number): # waveform for 4 sequential i-swap gates
        params = self._params
        if (number == 1):
            base = self._params["phi_base_level"]
        elif (number == 2):
            base = params["phi2z_base_level"]
            
        params = self._params
        t_starts = [params["start"], params["start"] + 90.1]
        t_finishes = t_starts.copy()
        for i in range (2):
            t_finishes[i] += params["duration"]
        #waveform = sum([self.waveform(t_starts[i],t_finishes[i]) for i in range(4)])
        waveform = self.waveform_iswap_zgate(number, t_starts[0], t_finishes[0]) +\
        self.waveform_iswap_zgate(number, t_starts[1], t_finishes[1]) - base
        #waveform = self.waveform_iswap_zgate(number, t_starts[1], t_finishes[1])
        return waveform
    
    def waveform_iswap_zgate(self, number, t_start = None, t_finish = None, t_zgate = None): #number - number of qubit
        params = self._params
        if t_start is None:
            t_start = params["start"]  
        if t_finish is None:
            t_finish =  params["start"] + params["duration"]
        if t_zgate is None:
            if (number == 1):
                t_zgate = params["t_zgate"]
            elif number == 2 :
                 t_zgate = params["t_zgate2"]
        if (number == 1):
            offset = self._params['phi_offset']
            offsetz = self._params["phiz_offset"]
            base = self._params["phi_base_level"]
            waveform1 = self._normalized_pulse(t_start, t_finish)*offset + base
            waveform_z = self._normalized_pulse(20 + t_finish, 20 + t_finish + t_zgate)*offsetz + base
            waveform = waveform1  + waveform_z - base
        elif (number == 2):
            offsetz = self._params["phi2z_offset"]
            base = params["phi2z_base_level"]
            waveform = self._normalized_pulse(20 + t_finish, 20 + t_finish + t_zgate)*offsetz + base
            #waveform = ones_like(self._Ts)*0.5
        return waveform
    
    
    def waveform_cphase (self, t_start = None, t_finish = None, offset = None):
        params = self._params
        if t_start is None:
            t_start = params["start_cphase"]  
        if t_finish is None:
            t_finish =  params["start_cphase"] + params["duration_cphase"]
        if offset is None:
            offset = self._params["phi_offset_cphase"]
        base = self._params["phi_base_level"]
        return self._normalized_pulse(t_start, t_finish)*offset+base     
        
    def plot_iswap_zgate(self, number):
        plot(self._Ts, self.waveform_iswap_zgate(number))
    
    def plot_iswap_zgate_second(self):
        plot(self._Ts, self.waveform_iswap_zgate_second())
        
    def plot_4iswap(self):
        plot(self._Ts, self.waveform_4iswap())   
    
    def plot_2iswap(self, number):
        plot(self._Ts, self.waveform_2iswap(number))   
        
    def plot(self):
        plot(self._Ts, self.waveform())
    def plot_ideal(self):
        plot(self._Ts,self.waveform1())

#класс для реализации массива трансмонов с произвольными связями
from single_transmon.Transmon import *
from qutip import *
from itertools import product
from numpy import zeros, array, where, ones_like, zeros_like
from matplotlib.pyplot import *
from scipy.sparse import coo_array, csr_matrix

class TransmonChain:

    def __init__(self, length, transmon_truncation=2,
                 transmon_diagonalization_dimension=15,
                 bose_hubbard_approximation = False):
        self._length = length #количество трансмонов в массиве
        self._transmon_truncation = transmon_truncation # число рассматриваемых уровней в трансмоне
        self._transmon_diagonalization_dimension = transmon_diagonalization_dimension # число учитывающихся зарядовых состояний
        self._bose_hubbard_approximation = bose_hubbard_approximation # использовать ли приближение Бозе-Хаббарда
        self._Ec = [] #Зарядовая энергия
        self._Ej = [] # Туннельная энергия
        self._J = [] # Константы взаимодействия
        self._d = [] # параметры асимметрии
        self._Omega = []
        self._omega = None
        self._gamma_phi = []
        self._gamma_rel = []
        self._phi = []
        self._c_ops = None
        self._drive_shape = 'gauss_hdrag' # форма импульса накачки
        self._transmons = [None] * self._length
        self._zero_op = tensor(*[qzero(self._transmon_truncation) for i in range(0, self._length)])
        self._identity_array = [qeye(self._transmon_truncation) for i in range(0, self._length)]

        self._RWA_driving_cache = {}
        self._RWA_subtrahend_cache = {}
        self._transmon_H_cache = [{} for _ in range(self._length)]
        self._interaction_cache = {}

        self._low_energy_states = []
        self._low_energy_state_indices = []
    
    def set_hd_amplitudes (self, hd_amplitude): #установка амплитуды корректирующих импульсов
        for tr in self._transmons:
            tr._hd_amplitude=hd_amplitude
    def set_e_ops(self): #выбор измеряемых операторов
        sigma_z_chain = []
        sigma_x_chain = []
        sigma_y_chain = []
        for i in range (self._length): 
            sigma_z = self._transmons[i].sz()
            sigma_x = self._transmons[i].sx()
            sigma_y = self._transmons[i].sy()
            oper_z = self._identity_array.copy()
            oper_z[i]=sigma_z
            oper_x = self._identity_array.copy()
            oper_x[i]=sigma_x
            oper_y = self._identity_array.copy()
            oper_y[i]=sigma_y
            sigma_z_chain.append(self.truncate_to_low_population_subspace(tensor(*oper_z)))
            sigma_x_chain.append(self.truncate_to_low_population_subspace(tensor(*oper_x)))
            sigma_y_chain.append(self.truncate_to_low_population_subspace(tensor(*oper_y)))
        self.e_ops = sigma_z_chain + sigma_x_chain + sigma_y_chain
        

        

    def clear_caches(self):
        self._RWA_driving_cache = {}
        self._RWA_subtrahend_cache = {}
        self._transmon_H_cache = [{} for _ in range(self._length)]
        self._interaction_cache = {}
        self._c_ops = None

    #переход в вычислительный базис
    def build_low_energy_kets(self, total_population, max_simultaneously_above_first_excited):
        self._low_energy_states = []
        self._low_energy_state_indices = []
        transmon_states = [list(range(self._transmon_truncation)) for i in range(self._length)]
        transmon_states = product(*transmon_states)
        
        idx = 0
        for idx, state_combination in enumerate(transmon_states):
            state_combination = array(state_combination)
            if sum(state_combination) <= total_population:
                if len(where(state_combination >= 2)[0]) <= max_simultaneously_above_first_excited:
                    self._low_energy_states.append(state_combination)
                    self._low_energy_state_indices.append(idx)
           
#         self._low_energy_states_mask = zeros([idx+1]*2, dtype=bool)
     
        self._low_energy_states_mask = csr_matrix((idx+1, idx+1), dtype=bool)
        self._low_energy_vectors_mask = zeros(idx+1, dtype=bool)
        for idx1 in self._low_energy_state_indices:
            self._low_energy_vectors_mask[idx1]=True
            for idx2 in self._low_energy_state_indices:
                self._low_energy_states_mask[idx1, idx2] = True
        
                    
        
        print("Total %d kets included" % len(self._low_energy_states))

    #построение гамильтониана в приближении вращающейся волны
    def build_H_RWA(self):
        return self.build_H_full() - self.build_RF_subtrahend()

    #построение полного гамильтониана системы
    def build_H_full(self, waveforms, params, rotations):
        H_chain = []
        omegas = []
        for i in range(0, self._length): #single transmon part
            H_chain += self._build_transmon_H_at_index(i, waveforms[i])
        for i in range(0, self._length): #transmons interaction
            evals1, evecs1 = self._transmons[i].H_diag_trunc_approx(self._phi[i]).eigenstates()
#             evals2, evecs2 = self._transmons[i+1].H_diag_trunc_approx(self._phi[i+1]).eigenstates()
            omega1 = (evals1[1]- evals1[0])
#             omega2 = (evals2[1]- evals2[0])
            omegas.append(omega1)
        for i in range(0, self._length):
            for j in range(0, i):
                H_chain += self.Hint_RF_RWA(j, i, omegas[i] - omegas[j])
        H_chain += self.build_RWA_driving(params,rotations)
        return H_chain
    #построение операторов коллапса
    def build_c_ops(self):  # building at 0 flux since no real dependence on anything
        if self._c_ops is not None:
            return self._c_ops

        self._c_ops = []

        for i in range(self._length):
            chain_operator_rel = self._identity_array.copy()
            chain_operator_rel[i] = self._transmons[i].c_ops(0)[0]
            chain_operator_deph = self._identity_array.copy()
            
            chain_operator_deph[i] = self._transmons[i].c_ops(0)[1]
            self._c_ops.append(self.truncate_to_low_population_subspace(tensor(*chain_operator_rel)))
            self._c_ops.append(self.truncate_to_low_population_subspace(tensor(*chain_operator_deph)))
#         return self._c_ops
    # обрезка до низкоэнергетического базиса
    def truncate_to_low_population_subspace(self, operator, obj_type = 'matrix'):
        if obj_type == 'matrix':
            return Qobj(operator.data[self._low_energy_states_mask].reshape((len(self._low_energy_state_indices), -1)))
        elif obj_type == 'vector':
            return Qobj(operator.data[self._low_energy_vectors_mask])

    #построение гамильтониана накачки в приближении вращающейся волны
    def build_RWA_driving(self, params, rotations):
        driving_operator = self._zero_op.copy()
        H_drive_full = []
        
        for i in range(0, self._length):
            dur, phi = rotations[i]
            drive_shape=self._transmons[i]._drive_shape
            if drive_shape=='gauss':
                #self._transmons[i]._drive_shape = 'gauss'
                ampl = dur # we interpret duration as amplitude
                Hdr, Hdr_coeff = self._transmons[i].Hdr_RF_RWA(ampl, 
                                                               params['start'] - params['gauss_pulse_duration'] -10,
                                                               params['gauss_pulse_duration'], 
                                                               phi)
                chain_operator = self._identity_array.copy()
                chain_operator[i] = Hdr
                chain_operator = tensor(*chain_operator)
                Hdr_trunc = self.truncate_to_low_population_subspace(chain_operator)
                H_drive_full +=[[Hdr_trunc,Hdr_coeff]] 
                
            elif drive_shape=='gauss_hdrag':
                ampl = dur   
                [Hdr1, Hdr_coeff1], [Hdr2, Hdr_coeff2] = self._transmons[i].Hdr_RF_RWA(ampl,
                                                               params['start'] - params['gauss_pulse_duration'] -10,
                                                               params['gauss_pulse_duration'], 
                                                               phi)
                chain_operator1 = self._identity_array.copy()
                chain_operator1[i] = Hdr1
                chain_operator1 = tensor(*chain_operator1)
                Hdr_trunc1 = self.truncate_to_low_population_subspace(chain_operator1)
                chain_operator2 = self._identity_array.copy()
                chain_operator2[i] = Hdr2
                chain_operator2 = tensor(*chain_operator2)
                Hdr_trunc2 = self.truncate_to_low_population_subspace(chain_operator2)
                H_drive_full +=[[Hdr_trunc1,Hdr_coeff1],[Hdr_trunc2,Hdr_coeff2]]
                 
            else:
                Hdr, Hdr_coeff = self._transmons[i].Hdr_RF_RWA(params['drive_amplitude'], 0, dur, phi) # Beginning of the rotation in the time 0ns
                chain_operator = self._identity_array.copy()
                chain_operator[i] = Hdr
                chain_operator = tensor(*chain_operator)
                Hdr_trunc = self.truncate_to_low_population_subspace(chain_operator)
                H_drive_full +=[[Hdr_trunc,Hdr_coeff]] 
            
        return H_drive_full

    # построение гамильтониана одного трансмона
    def _build_transmon_H_at_index(self, i, waveform):
        
        chain_operator = self._identity_array.copy()
        H1 = self._identity_array.copy()
        H2 = self._identity_array.copy()
        Huu = self._identity_array.copy() 
        
        evals1, evecs1 = self._transmons[i].H_diag_trunc_approx(self._phi[i]).eigenstates()
        freq1 = (evals1[1]- evals1[0])/2/pi
        Huu[i] = self._transmons[i].H_uu(freq1)*(-1)
        Huu = tensor(*Huu)
        Huu_trunc = self.truncate_to_low_population_subspace(Huu)
        
        if self._transmon_truncation == 2: #Случай двухуровневых трансмонов
            [H1[i],H1_coeff] = self._transmons[i].H_td_diag_trunc_approx(waveform)
            H1 = tensor(*H1)
            H1_trunc = self.truncate_to_low_population_subspace(H1)
            return [[H1_trunc,H1_coeff]] + [[Huu_trunc, ones_like(H1_coeff)]]
        
        elif self._transmon_truncation == 3: # Для трёхуровневых трансмонов
            [H1[i],H1_coeff], [H2[i], H2_coeff] = self._transmons[i].H_td_diag_trunc_approx(waveform)            
            H1 = tensor(*H1)
            H2 = tensor(*H2)           
            
            H1_trunc = self.truncate_to_low_population_subspace(H1)
            H2_trunc = self.truncate_to_low_population_subspace(H2)
        
            return [[H1_trunc,H1_coeff]] + [[H2_trunc, H2_coeff]] + [[Huu_trunc, ones_like(H1_coeff)]]

    #визуализация динамики массива кубитов
    def plot_chain_dynamic(self, result):
        Ts = self._Ts
        fig, axes = subplots (3,1, figsize = (10,8),sharex=True)
        labels = [' ', ' ', ' ' ]
#         qubits = {0:'I', 1:'II', 2:'III', 3:'IV', 4:'V', 5:'VI', 6:'VII', 7:'VIII', 
#                   8:'IX', 9:'X', 10:'XI', 11:'XII'}
        qubits = {0:'0', 1:'1', 2:'2', 3:'3', 4:'4', 5:'5', 6:'6', 7:'7', 8:'8', 
               9:'9', 10:'10', 11:'11', 12:'12', 13:'13', 14:'14', 15:'15'}
        for ind in range (3):            
            for i in range (self._length):
                axes[ind].plot(Ts, result.expect[i + ind*self._length], label = labels[ind] + qubits[i])       
        
        for ax in axes:
            ax.set_ylim((-1.05,1.05))
            ax.legend(loc='right', prop={'size': 5})
            ax.grid()
        axes[0].set_ylabel(r'$\langle \sigma_{z} \rangle$',fontsize = 10)
        axes[1].set_ylabel(r'$\langle \sigma_{x} \rangle$',fontsize = 10) 
        axes[2].set_ylabel(r'$\langle \sigma_{y} \rangle$',fontsize = 10) 
        axes[2].set_xlabel('$T$, нс',fontsize = 15)
        return fig, axes

    #Двумерная визуализация динамики массива кубитов
    def plot_chain_dynamic_2D(self, result, chosen_qubits):
        fig, axes = subplots (1,3, figsize = (10,5),sharey=True)
        data = np.array(result.expect).T
        number_of_qubits = self._length
        for i in range(3):
            ax = axes[i]
            image = ax.imshow(data[:, i*number_of_qubits + chosen_qubits], cmap = 'RdBu_r',  aspect = 'auto', origin = 'lower',
                                 vmin = -1, vmax = 1,
                                 extent = [0, len(chosen_qubits), self._Ts[0], self._Ts[-1]],
                                 interpolation='none', rasterized=True)
            cb = fig.colorbar(image, ax=ax)
            xticks = np.arange(len(chosen_qubits))+0.5
            xlabels = chosen_qubits

            ax.set_xticks(np.array(xticks))
            ax.set_xticklabels(xlabels, fontsize=10)
            ax.set_xlabel('Номер кубита')
        axes[0].set_ylabel('T, ns',fontsize = 12)
        axes[0].set_title(r'$\langle \sigma_{z} \rangle$',fontsize = 10)
        axes[1].set_title(r'$\langle \sigma_{x} \rangle$',fontsize = 10) 
        axes[2].set_title(r'$\langle \sigma_{y} \rangle$',fontsize = 10)
        return fig, axes

    #построение гамильтониана взаимодействия кубитов
    def Hint_RF_RWA(self, i, j, offset):
        chain_operator1 = self._identity_array.copy()      
        chain_operator2 = self._identity_array.copy()
        op1 = self._transmons[i].n(self._phi[i]).data.copy()
        op2 = self._transmons[j].n(self._phi[j]).data.copy()
        # zeroing triangles
        #print (op1)
        #print (op2)
        for k in range(op1.shape[0]):
            for l in range(op1.shape[1]):
                if k < l:
                    op1[k, l] = 0
                if k > l:
                    op2[k, l] = 0
                    
        
        chain_operator1[i] = Qobj(op1, dims = [[self._transmon_truncation],[self._transmon_truncation]])
        chain_operator1[j] = Qobj(op2, dims = [[self._transmon_truncation],[self._transmon_truncation]])
        half = tensor(*chain_operator1) 
        #print (half)
        half_trunc = self.truncate_to_low_population_subspace(half)
        return [[self._J[i,j] * half_trunc, "exp(-1j*(%f)*t)"%offset],[self._J[i,j] * half_trunc.dag(), "exp(1j*(%f)*t)"%offset]]

    #создание каждого из трансмонов
    def build_transmons(self):
        for i in range(self._length):
            self._transmons[i] = Transmon(self._Ec[i], self._Ej[i], self._d[i],
                                          self._gamma_rel[i], self._gamma_phi[i],
                                          self._transmon_diagonalization_dimension,
                                          self._transmon_truncation, i,
                                          self._bose_hubbard_approximation)

    def get_length(self):
        return self._length

    #установка зарядовой энергии
    def set_Ec(self, Ec):
        try:
            assert len(Ec) == self._length
            self._Ec = Ec
        except TypeError:
            self._Ec = [Ec] * self._length
            print("Setting all Ec to be equal")
        except AssertionError:
            raise ValueError("Length of Ec is not equal to the number of transmons")

    #установка туннельной энергии
    def set_Ej(self, Ej):
        try:
            assert len(Ej) == self._length
            self._Ej = Ej
        except TypeError:
            self._Ej = [Ej] * self._length
            print("Setting all Ej to be equal")
        except AssertionError:
            raise ValueError("Length of Ej is not equal to the number of transmons")

    #установка силы связи между трансмонами
    def set_J(self, J):
        try:
            assert len(J) == self._length
            self._J = J
#         except TypeError:
#             self._J = [J] * self._length
#             print("Setting all J to be equal")
        except AssertionError:
            raise ValueError("Length of J is not equal to the number of transmons")

    def set_Omega(self, Omega):
        try:
            assert len(Omega) == self._length
            self._Omega = Omega
        except TypeError:
            self._Omega = [Omega] * self._length
            print("Setting all Omega to be equal")
        except AssertionError:
            raise ValueError("Length of Omega is not equal to the number of transmons")

    def set_omega(self, omega):
        self._omega = omega

    #установка параметров асимметрии
    def set_asymmetry(self, d):
        try:
            assert len(d) == self._length
            self._d = d
        except TypeError:
            self._d = [d] * self._length
            print("Setting all d to be equal")
        except AssertionError:
            raise ValueError("Length of d is not equal to the number of transmons")

    #установка констант релаксации и дефазировки
    def set_gamma_rel(self, gamma_rel):
        try:
            assert len(gamma_rel) == self._length
            self._gamma_rel = gamma_rel
        except TypeError:
            self._gamma_rel = [gamma_rel] * self._length
            print("Setting all gamma_rel to be equal")
        except AssertionError:
            raise ValueError("Length of gamma_rel is not equal to the number of transmons")

    def set_gamma_phi(self, gamma_phi):
        try:
            assert len(gamma_phi) == self._length
            self._gamma_phi = gamma_phi
        except TypeError:
            self._gamma_phi = [gamma_phi] * self._length
            print("Setting all gamma_phi to be equal")
        except AssertionError:
            raise ValueError("Length of gamma_phi is not equal to the number of transmons")

    def set_phi(self, phi):
        try:
            assert len(phi) == self._length
            self._phi = phi
        except TypeError:
            self._phi = [phi] * self._length
            print("Setting all fluxes to be equal")
        except AssertionError:
            raise ValueError(
                "Length of fluxes is not equal to the number of transmons: " + str(phi))

#подключение необходимых библиотек
import numpy as np
from numpy import random, linspace, real, imag, abs, angle, exp
from qutip import ket2dm, tensor, basis
from matplotlib.pyplot import *

# Класс для моделирования считывающего резонатора
class ReadoutResonator:

    def __init__(self, f_res, Ql, Qc, line_att=0.0345, phi=0.5, noise_sigma=1e-4, delay=0., alpha = 0):
        self._f_res = f_res # частота резонатора
        self._Ql = Ql # добротность резонатора в присутствии нагрузки
        self._Qc = Qc # добротность из-за связи с измерительной линией
        self._line_att = line_att
        self._phi = phi
        self._noise_sigma = noise_sigma
        self._delay = delay
        self._alpha = alpha

    #установка параметров кубитов
    def set_qubit_parameters(self, g1, g2, f1, f2, alpha_1, alpha_2):
        self._g1 = g1
        self._g2 = g2
        self._f1 = f1
        self._f2 = f2
        self._alpha_1 = alpha_1
        self._alpha_2 = alpha_2

    def set_noise_sigma(self, sigma):
        self._noise_sigma = sigma

    #вычисление параметра рассеяния S_21
    def S_param(self, f, fr_shift):
        try:
            size = len(f)
        except TypeError:
            size = 1

        noise = 1 * random.normal(size=size, scale=self._noise_sigma / 2) + \
                1j * random.normal(size=size, scale=self._noise_sigma / 2)

        env_term = self._line_att * np.exp(-2 * np.pi * 1j * self._delay * f+1j*self._alpha)
        own_term = exp(1j * self._phi) * (self._Ql / self._Qc) / (
                1 + 2 * 1j * self._Ql * ((f / (self._f_res - fr_shift)) - 1))
        result = env_term * (1 - own_term) + noise
        return result if size > 1 else result[0]
    # вычисление дисперсионных сдвигов
    def get_dipsersive_shifts(self):

        chi_1_01 = -self._g1 ** 2 / (self._f1 - self._f_res)
        chi_1_12 = -2 * self._g1 ** 2 / (self._f1 - self._alpha_1 - self._f_res)
        chi_1_23 = -3 * self._g1 ** 2 / (self._f1 - 2 * self._alpha_1 - self._f_res)
        chi_2_01 = -self._g2 ** 2 / (self._f2 - self._f_res)
        chi_2_12 = -2 * self._g2 ** 2 / (self._f2 - self._alpha_2 - self._f_res)
        chi_2_23 = -3 * self._g2 ** 2 / (self._f2 - 2 * self._alpha_2 - self._f_res)

        shift_00 = 0
        shift_10 = 2 * chi_1_01 - chi_1_12
        shift_20 = chi_1_01 + chi_1_12 - chi_1_23
        shift_01 = 2 * chi_2_01 - chi_2_12
        shift_02 = chi_2_01 + chi_2_12 - chi_2_23
        shift_11 = shift_01 + shift_10
        shift_21 = shift_20 + shift_01
        shift_12 = shift_10 + shift_02
        shift_22 = shift_20 + shift_02

        return [[shift_00, shift_01, shift_02],
                [shift_10, shift_11, shift_12],
                [shift_20, shift_21, shift_22]]

    #оператор измерения
    def measurement_operator(self, f, shifts):
        N = len(shifts)
        return sum([sum(
            [ket2dm(tensor(basis(N, i), basis(N, j))) * self.S_param(f, shift) for j, shift in
             enumerate(row)]) for
             i, row in enumerate(shifts)])

    #отрисовка параметров пропускания
    def plot_all(self, fs):
        if fs is None:
            fs = linspace(self._f_res - 10e-3, self._f_res + 10e-3, 201)
        fig, axes = subplots(nrows=2, ncols=2, figsize=(15, 5))
        axes = axes.ravel()
        shifts = self.get_dipsersive_shifts()

        for idx, digest in enumerate([abs, angle, real, imag]):
            for i, row in enumerate(shifts):
                for j, shift in enumerate(row):
                    axes[idx].plot(fs, digest(self.S_param(fs, shift)), label="%d%d" % (i, j))
        legend()

    def plot(self, shift=0, fs=None, digest=real):
        if fs is None:
            fs = linspace(self._f_res - 10e-3, self._f_res + 10e-3, 201)
        plot(fs, digest(self.S_param(fs, shift)))

#Подключение необходимых библиотек
# %matplotlib notebook
import matplotlib.pyplot as plt
from importlib import reload
import two_transmons.DoubleTransmonSystem
from two_transmons.DoubleTransmonSystem import *
import two_transmons.VacuumRabiSimulation
from two_transmons.VacuumRabiSimulation import *
import single_transmon.Transmon
from single_transmon.Transmon import *
from ReadoutResonator import *
from qutip import *
from tqdm.notebook import tqdm
from two_transmons.ZPulse import *
from importlib import reload
import two_transmons.ParallelCalculations
from two_transmons.ParallelCalculations import *
from two_transmons.CalcTime import *
from scipy.linalg import cholesky
from p_tqdm import p_map, p_imap
import time
import numpy as np
from numpy import pi
from numpy import dot
from copy import deepcopy
from transmon_chain.transmon_chain import *
from two_transmons.Vacuum_rabi_windows import *
import warnings
import pickle
from IPython.display import Image, display
warnings.filterwarnings("ignore")

#Задание параметров трансмонов в цепочке (в данном случае они берутся из результатов эксперимента)
Nc = 7
N_trunc = 2

qubit_parameters = { 'I' : {'fge_max': 5.763,
                            'd':0.696,
                            'anharmonicity':0.13,
                            'fge_min':4.81
                           },
                    'II' : {'fge_max': 5.82,
                            'd':0.620,
                            'anharmonicity':0.132,
                            'fge_min':4.583
                           },
                    'III' : {'fge_max': 5.735,
                            'd':0.673,
                            'anharmonicity':0.110,
                            'fge_min':4.703
                           }
                   }
Ec1 = qubit_parameters['I']['anharmonicity']*2*pi
Ej1 = (qubit_parameters['I']['fge_max']**2)/8/qubit_parameters['I']['anharmonicity']*2*pi
d1 = qubit_parameters['I']['d']
Ec2 = Ec3 = Ec1
Ej2 = Ej3 = Ej1
d2 = d3 = d1

number_of_qubits = 16
Ecc = 1/80.0 * 2 * np.pi/2 # derived from experiment

T1_1, T2_1 = 10e3, 5e3
T1_2, T2_2 = 10e3, 5e3
T1_3, T2_3 = 10e3, 5e3

tr1 = Transmon(Ec1, Ej1, d1, 1/T1_1, 1/T2_1, Nc, N_trunc, 1)
tr2 = Transmon(Ec2, Ej2, d2, 1/T1_2, 1/T2_2, Nc, N_trunc, 2)
tr3 = Transmon(Ec3, Ej3, d3, 1/T1_3, 1/T2_3, Nc, N_trunc, 3)
tr4 = Transmon(Ec1, Ej1, d1, 1/T1_1, 1/T2_1, Nc, N_trunc, 4)
tr5 = Transmon(Ec2, Ej2, d2, 1/T1_2, 1/T2_2, Nc, N_trunc, 5)
tr6 = Transmon(Ec3, Ej3, d3, 1/T1_3, 1/T2_3, Nc, N_trunc, 6)
tr7 = Transmon(Ec1, Ej1, d1, 1/T1_1, 1/T2_1, Nc, N_trunc, 7)
tr8 = Transmon(Ec2, Ej2, d2, 1/T1_2, 1/T2_2, Nc, N_trunc, 8)
tr9 = Transmon(Ec3, Ej3, d3, 1/T1_3, 1/T2_3, Nc, N_trunc, 9)
tr10 = Transmon(Ec1, Ej1, d1, 1/T1_1, 1/T2_1, Nc, N_trunc, 10)
tr11 = Transmon(Ec2, Ej2, d2, 1/T1_2, 1/T2_2, Nc, N_trunc, 11)
tr12 = Transmon(Ec3, Ej3, d3, 1/T1_3, 1/T2_3, Nc, N_trunc, 12)
tr13 = Transmon(Ec1, Ej1, d1, 1/T1_1, 1/T2_1, Nc, N_trunc, 13)
tr14 = Transmon(Ec2, Ej2, d2, 1/T1_2, 1/T2_2, Nc, N_trunc, 14)
tr15 = Transmon(Ec3, Ej3, d3, 1/T1_3, 1/T2_3, Nc, N_trunc, 15)
tr16 = Transmon(Ec1, Ej1, d1, 1/T1_1, 1/T2_1, Nc, N_trunc, 16)

farg=cook_farg(20,0.2)
params=farg[0]['params']
#Создание сетки по времени
Ts = linspace(0, params["finish"], params['t_points'])

### 16-кубитная схема

display(Image(filename='Pictures/16Q_design.png'))

#Создание решётки трансмонов
chain = TransmonChain(number_of_qubits, transmon_truncation = N_trunc)
chain.set_phi(list(params['phi_base_levels'].values()))
chain._transmons = [tr1, tr2,tr3,tr4, tr5, tr6, tr7, tr8, tr9, tr10, tr11, tr12, tr13, tr14, tr15, tr16]
# Определение структуры решётки трансмонов
J = np.zeros((number_of_qubits, number_of_qubits))
for i in [2, 3, 4, 6, 7, 8, 10, 11, 12]:
    J[i, i + 3] = Ecc
for i in [0, 2, 3, 5, 6, 7, 9, 10, 11, 13, 14]:
    J[i, i + 1] = Ecc
J[0, 2] = Ecc
J[1, 3] = Ecc
chain.set_J(J)

chain.build_low_energy_kets(2,1)
chain._Ts = Ts
#инициализацияя начального состояния
rho0 = [ket2dm(basis(N_trunc,0))]*number_of_qubits
psi0 = [basis(N_trunc,0)]*number_of_qubits
rho0 = tensor(*rho0)
psi0 = tensor(*psi0)
rho0 = chain.truncate_to_low_population_subspace(rho0, obj_type = 'matrix')
psi0 = chain.truncate_to_low_population_subspace(psi0, obj_type = 'vector')

chain.rho0 = rho0
chain.psi0 = psi0
chain.set_e_ops()
chain.build_c_ops()

### Построение спектра 16-кубитной системы

phi_arr = list(params['phi_base_levels'].values())# [0.5, 0.0, 0.5, 0.0, 0.5, 0.0, 0.5, 0.0, 0.5, 0.0, 0.0, 0.5]
#Функция построения спектра системы в зависимости от потока через выбранный кубит
def plot_spectrum(qubit, points, phi_range = [0,1],
                  phis = [0.05, 0.1, 0.15, 0.2, 0.25, 0.3, 0.35, 0.4, 0.45, 0.5, 0.52, 0.62, 0.67, 0.72, 0.77, 0.82]):
    phi_arr = deepcopy(phis)
    levels = []
    #инициализация значений потока через данный кубит
    phi_range =linspace(phi_range[0], phi_range[1], points)
    for phi in tqdm(phi_range):
        waveforms_const = []
        phi_arr[qubit] = phi
        chain.set_phi(phi_arr)
        for idx in range(chain._length):
            waveforms_const.append(ones_like(Ts)*phi_arr[idx])
        #построение Гамильтониана системы    
        H_full = chain.build_H_full(waveforms_const, 
                                     params, [[0,0]]*number_of_qubits)
        H_const = chain.truncate_to_low_population_subspace(chain._zero_op)
        for i in range (number_of_qubits*2):
            if (i%2 == 0):
                H_const+=H_full[i][0]*H_full[i][1][0]
        for i in range(2*number_of_qubits, len(H_full),1):
            H_const+=H_full[i][0]
            
        #Нахождение собственных значений гамильтониана - энергетических уровней системы
        evals, evecs = H_const.eigenstates()
        levels.append((evals-evals[0])/2/pi)
    #построение графика
    plt.figure()
    plt.plot(phi_range, array(levels)[:, 1:17])
    plt.ylabel(r'$\nu$, ГГц')
    plt.xlabel(r'Относительный магнитный поток $\frac{\Phi}{\Phi_{0}}$')
#     plt.title(f'Спектр кубита №{qubit}')
    plt.grid()
    chain.set_phi(list(params['phi_base_levels'].values()))

phis = np.linspace(0.05, 0.45, 16)
plot_spectrum(1, 101, phis = phis)
plt.savefig('Pictures/Spectrum16Q.png', bbox_inches = 'tight')
plt.savefig('Pictures/Spectrum16Q.pdf', bbox_inches = 'tight')

plot_spectrum(1,21, phi_range = [0.03, 0.08], phis=phis)
plt.ylim([5.58, 5.66])
plt.savefig('Pictures/Spectrum_16Q_anticross_01.png', bbox_inches='tight')
plt.savefig('Pictures/Spectrum_16Q_anticross_01.pdf', bbox_inches='tight')

plot_spectrum(1,51, phi_range = [0.1, 0.16], phis=phis)
plt.ylim([5.47, 5.56])
plt.savefig('Pictures/Spectrum_16Q_anticross_13.png', bbox_inches='tight')
plt.savefig('Pictures/Spectrum_16Q_anticross_13.pdf', bbox_inches='tight')

plot_spectrum(1,51, phi_range = [0.07, 0.13], phis=phis)
plt.ylim([5.565, 5.575])
plt.savefig('Pictures/Spectrum_16Q_anticross_12.png', bbox_inches='tight')
plt.savefig('Pictures/Spectrum_16Q_anticross_12.pdf', bbox_inches='tight')

### Однокубитные операции

#Определение потоков через каждый из кубитов
waveforms, drive_amplitudes = [], []
params['start'] = 10
for qubit in range(16):    
    waveforms.append(ones_like(Ts)*params['phi_base_levels'][qubit])
    if qubit == 0:
        drive_amplitudes.append([params['finish'],0])                           
    else:
        drive_amplitudes.append([0,0])
        
H_full_test = chain.build_H_full(waveforms, params, drive_amplitudes)

result = mesolve(H_full_test, chain.psi0, Ts, 
                 e_ops = chain.e_ops,
                 progress_bar=True,options=Options(nsteps = 20000, store_states = False, max_step = 1e-1))

fig,axes = chain.plot_chain_dynamic(result)
fig.savefig('Pictures/1Q_operations.pdf', bbox_inches='tight')
fig.savefig('Pictures/1Q_operations.png', bbox_inches='tight')

### Блоховские осцилляции

#установка длительности эксперимента
params['finish']=400
Ts = linspace(0, params["finish"], params['t_points'])
chain._Ts = Ts

# выбор задействованных кубитов
chosen_qubits = np.array([1, 0, 2, 5, 6, 9, 10, 13, 14, 15, 12])
# Выбор рабочих частот каждого из трансмонов
freq_offsets = np.arange(0.0, 0.041, 0.008)
upper_sws = 5.4
lower_sws = 4.7
exc_qubit = 1

def simulate_BOs (chain, chosen_qubits, exc_qubit, freq_offset):
    psi0 = [basis(2,0)]*number_of_qubits
    psi0[exc_qubit] = basis(2,1) # задаём начальное состояние 1го кубита
    psi0 = tensor(*psi0)
    psi0 = chain.truncate_to_low_population_subspace(psi0, obj_type = 'vector')

      
    energies = [lower_sws for i in range(number_of_qubits)]

    for ind, qubit in enumerate(chosen_qubits):
        energies[qubit] = upper_sws - ind*freq_offset
        
    phis_wsl = [tr1.inverse_transmon_spectrum(energy) for energy in energies]
    
    #Определение потоков через каждый из кубитов
    waveforms, drive_amplitudes = [], []
    for qubit in range(16):    
        waveforms.append(ones_like(Ts)*phis_wsl[qubit]) 
        drive_amplitudes.append([0,0])  
    params['start']=0                         
    signal = ZPulse(Ts,params)                      

    #построение гамильтониана системы
    H_full_test = chain.build_H_full(waveforms, params, drive_amplitudes )

    # Решение уравнения Шредингера для системы
    result = mesolve(H_full_test, psi0, Ts, 
                     c_ops = [],e_ops = chain.e_ops,
                     progress_bar=True,options=Options(nsteps = 20000, store_states = True, max_step = 8e-1))
    
    return chain, result

# Проведение расчётов для нескольких значений отстройки по частоте
results = []
for freq_offset in freq_offsets:
    chain, result = simulate_BOs (chain, chosen_qubits, exc_qubit, freq_offset)
    results.append(result)

# Визуализация результата
fig, axes = plt.subplots (2,3, figsize = (10,7),sharey=True, sharex=True)
letters = ['а', 'б', 'в', 'г', 'д', 'е']
for idx, ax in enumerate(axes.flatten()):
    result = results[idx]
    data = np.array(result.expect).T
    image = ax.imshow(data[:, chosen_qubits], cmap = 'RdBu_r',  aspect = 'auto', origin = 'lower',
                      vmin = -1, vmax = 1,
                          extent = [0, len(chosen_qubits), chain._Ts[0], chain._Ts[-1]],
                         interpolation='none', rasterized=True)
    cb = fig.colorbar(image, ax=ax)
    xticks = np.arange(len(chosen_qubits))+0.5
    xlabels = chosen_qubits
    ax.text((len(chosen_qubits)/2)*1.3, 0.9*chain._Ts[-1], f'F={freq_offsets[idx]*1000} МГц',
                        ha="center", va="center", color="w")
    ax.text(0.0, 1.05*chain._Ts[-1], f'({letters[idx]})',ha="center", va="center", color="black")
    

for ax in axes[1]:
    ax.set_xticks(np.array(xticks))
    ax.set_xticklabels(xlabels, fontsize = 9)
    ax.set_xlabel('Номер кубита')
axes[0,0].set_ylabel('T, нс')
axes[1,0].set_ylabel('T, нс')
# Сохранение результата
fig.savefig(f'Pictures/Bloch_oscillations_11Q.pdf', bbox_inches='tight')
fig.savefig(f'Pictures/Bloch_oscillations_11Q.png', bbox_inches='tight')

Pmax_allq = []
for idx, qubit in enumerate(chosen_qubits):
    Pmax_q=[]
    for idx1, result in enumerate(results):
        Pmax_q.append(np.max(result.expect[qubit])/2+0.5)
    Pmax_allq.append(Pmax_q)

fig, ax = plt.subplots (1,1, figsize = (10,5))

for qubit, probabilities in zip(chosen_qubits, Pmax_allq):
    ax.plot(freq_offsets*1000, probabilities, label = f'Q{qubit}' )
ax.legend()
ax.grid()
plt.yscale('log')
ax.set_xticks(freq_offsets*1000)
ax.set_xlabel('F, Мгц', fontsize = 12)
ax.set_ylabel(r'$P_{max}$', fontsize = 12)
fig.savefig(f'Pictures/Bloch_oscillations_1d.pdf', bbox_inches='tight')
fig.savefig(f'Pictures/Bloch_oscillations_1d.png', bbox_inches='tight')

### Параметрическая перестройка частоты кубитов

params['finish']=1000
Ts = linspace(0, params["finish"], params['t_points'])
chain._Ts = Ts

def simulate_parametric_drive_BOs (chain, chosen_qubits, exc_qubit, freq_offset, Fd, K, phase=0 ):
    waveforms, drive_amplitudes = [], []
    energies = [lower_sws for i in range(number_of_qubits)]
    delta_omegas = [zeros_like(Ts) for energy in energies]
    for ind, qubit in enumerate(chosen_qubits):
        #### case from the article
        energies[qubit] = upper_sws - ind*freq_offset 
        delta_omega = (len(chosen_qubits)//2 - ind)*K*freq_offset*np.cos(2*np.pi*Fd*chain._Ts + phase)
        delta_omegas[qubit] = delta_omega

    phis_parametric_drive = [chain._transmons[0].inverse_transmon_spectrum(energy+delta_omega) 
                             for energy, delta_omega in zip (energies, delta_omegas)]
    for qubit in range(16):    
        waveforms.append(phis_parametric_drive[qubit]) 
        drive_amplitudes.append([0,0]) 

    psi0 = [basis(2,0)]*number_of_qubits
    psi0[exc_qubit] = basis(2,1) # задаём начальное состояние 1го кубита
    psi0 = tensor(*psi0)
    psi0 = chain.truncate_to_low_population_subspace(psi0, obj_type = 'vector')
    #Определение потоков через каждый из кубитов

    params['start']=0                         
    signal = ZPulse(Ts,params)                      

    #построение гамильтониана системы
    H_full_test = chain.build_H_full(waveforms, params, drive_amplitudes )

    # Решение уравнения Шредингера для системы
    result = mesolve(H_full_test, psi0, Ts, 
                     c_ops = [],e_ops = chain.e_ops,
                     progress_bar=True,options=Options(nsteps = 20000, store_states = True, max_step = 8e-1))
    return chain, result

#### Форма управляющих импульсов

freq_offset=Fd=0.04
K = 0.2

fig, axes = plt.subplots(2,1, figsize= (7,7) )
energies = [lower_sws*ones_like(chain._Ts) for i in range(number_of_qubits)]
delta_omegas = [zeros_like(Ts) for energy in energies]
for ind, qubit in enumerate(chosen_qubits):
    ##
    energies[qubit] = upper_sws - ind*freq_offset
    delta_omega = (len(chosen_qubits)//2 - ind)*K*freq_offset*np.cos(2*np.pi*Fd*chain._Ts)
    ##
    
#     energies[qubit] = upper_sws - abs((len(chosen_qubits)//2 - ind))*freq_offset 
#     delta_omega = -abs((len(chosen_qubits)//2 - ind))*K*freq_offset*np.cos(2*np.pi*Fd*chain._Ts)
    ###
    delta_omegas[qubit] = delta_omega
    
    
    energies[qubit] = energies[qubit] + delta_omega
    
phis_parametric_drive = [chain._transmons[0].inverse_transmon_spectrum(energy+delta_omega) 
                             for energy, delta_omega in zip (energies, delta_omegas)]
energies = np.array(energies)
for qubit in chosen_qubits:
    axes[0].plot(chain._Ts, energies[qubit], label = f'Q{qubit}')
    axes[1].plot(chain._Ts, phis_parametric_drive[qubit], label = f'Q{qubit}')
for ax in axes:
    ax.grid()
    ax.legend(loc='upper right', fontsize=8 )
    ax.set_xlabel('T, нс')
axes[0].set_ylabel(r'$\nu$, ГГЦ')
axes[0].text(-75.0, 5.54, f'({letters[0]})',ha="center", va="center", color="black", fontsize=12)
axes[1].text(-75.0, 0.49, f'({letters[1]})',ha="center", va="center", color="black", fontsize=12)
axes[1].set_ylabel(r'Относительный магнитный поток, $\frac{\Phi}{\Phi_0}$')
fig.savefig(f'Pictures/Waveforms.pdf', bbox_inches='tight')
fig.savefig(f'Pictures/Waveforms.png', bbox_inches='tight')

chosen_qubits = np.array([1,0,2,5,6,9,10,13,14,15,12])
freq_offset = 0.040
Fd = 0.040
exc_qubit = 9
upper_sws = 5.4
lower_sws = 4.7
results = []
K=0.2
Ks = np.arange(0.0, 0.55, 0.1)
for K in Ks:
    chain, result = simulate_parametric_drive_BOs(chain, chosen_qubits, exc_qubit, freq_offset, Fd, K, phase=0 )
    results.append(result)

#### 2D график с фиксированным F при различных K
fig, axes = plt.subplots (2,3, figsize = (10,7),sharey=True, sharex=True)
for idx, ax in enumerate(axes.flatten()):
    result = results[idx]
    data = np.array(result.expect).T
    image = ax.imshow(data[:, chosen_qubits], cmap = 'RdBu_r',  aspect = 'auto', origin = 'lower',
                      vmin = -1, vmax = 1,
                          extent = [0, len(chosen_qubits), chain._Ts[0], chain._Ts[-1]],
                         interpolation='none', rasterized=True)
    cb = fig.colorbar(image, ax=ax)
    xticks = np.arange(len(chosen_qubits))+0.5
    xlabels = chosen_qubits
    K = Ks[idx]
    ax.text((len(chosen_qubits)/2)*1.5, 0.9*chain._Ts[-1], f'K={K:.2f}',
                        ha="center", va="center", color="w")
    ax.text(0.0, 1.05*chain._Ts[-1], f'({letters[idx]})',ha="center", va="center", color="black")
for ax in axes[1]:
    ax.set_xticks(np.array(xticks))
    ax.set_xticklabels(xlabels, fontsize = 9)
    ax.set_xlabel('Номер кубита')
axes[0,0].set_ylabel('T, нс')
axes[1,0].set_ylabel('T, нс')
t_finish = params['finish']
fig.savefig(f'Pictures/Parametric_drive_2D_F={freq_offset*1000}MHz_{t_finish//1000}us.pdf', bbox_inches='tight')
fig.savefig(f'Pictures/Parametric_drive_2D_F={freq_offset*1000}MHz_{t_finish//1000}us.png', bbox_inches='tight')

### Осцилляции размера волнового пакета с фиксированным F при различных K
fig, axes = plt.subplots (2,3, figsize = (10,5),sharey=True, sharex=True)
spatial_extents = []
for idx, ax in enumerate(axes.flatten()):
    result = results[idx]
    data = np.array(result.expect).T
    spatial_extent = zeros_like(data[:, 0])
    targ_qubit= len(chosen_qubits)//2
    for idx1, qubit in enumerate(chosen_qubits):
        spatial_extent+=(data[:,qubit]+1)/2*abs(idx1-targ_qubit)
    spatial_extents.append(spatial_extent)
    K = Ks[idx]
    ax.plot(chain._Ts, spatial_extent)
    ax.text(0.1*chain._Ts[-1], 2.35, f'K={K:.2f}',
                        ha="center", va="center", color="black")
    ax.text(0.0, 2.65, f'({letters[idx]})',ha="center", va="center", color="black")
    ax.grid()
for ax in axes[:, 0]:
    ax.set_ylabel('D, кубитов')
for ax in axes[1]:
    ax.set_xlabel('T, нс')
fig.savefig(f'Pictures/Parametric_drive_1D_F={freq_offset*1000}MHz_{t_finish//1000}us.pdf', bbox_inches='tight')
fig.savefig(f'Pictures/Parametric_drive_1D_F={freq_offset*1000}MHz_{t_finish//1000}us.png', bbox_inches='tight')

import pickle
to_save={'K':K, 'results': results, 'Fs':freq_offsets, 'Fd': Fd}
with open (f'Parametric_drive_2D_K={K}_{t_finish}us.pkl', 'wb') as f:
    pickle.dump(to_save, f)

import pickle
to_save={'Ks':Ks, 'results': results, 'F':freq_offset, 'Fd': Fd}
with open (f'Parametric_drive_2D_F={freq_offset*1000}MHz_{t_finish}us.pkl', 'wb') as f:
    pickle.dump(to_save, f)

#### Преобразование Фурье полученных осцилляций

fig,ax = plt.subplots(1,1, figsize = (6,4))
for K, spat_ext in zip (Ks[::],spatial_extents[::]):
    sig = np.fft.fftshift(np.fft.fft(spat_ext, n = 5*chain._Ts.shape[0])/len(chain._Ts))
    freq = np.fft.fftshift(np.fft.fftfreq(5*chain._Ts.shape[0], chain._Ts[1]-chain._Ts[0]))
    ax.plot(freq*1000, abs(sig), label = f'K={K:.2f}')
ax.set_xlim(0, 50)
ax.set_ylim(0,1.0)
ax.legend(loc='upper right')
ax.grid()
ax.set_xlabel(r'$\nu$, МГц')
ax.set_ylabel(r'$D(\nu)$, у.е.')
fig.savefig(f'Pictures/Parametric_drive_FFT_F={freq_offset*1000}MHz_{t_finish//1000}us.pdf', bbox_inches='tight')
fig.savefig(f'Pictures/Parametric_drive_FFT_F={freq_offset*1000}MHz_{t_finish//1000}us.png', bbox_inches='tight')

| Лист регистрации изменений |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Изм. | Номера листов (страниц) |  |  |  | Всего листов (страниц) в докум. | Номер докум. | Входящий номер сопроводительного докум. и дата | Подп. | Дата |
|  | изменен-ных | заменен-ных | новых | аннули-рован-ных |  |  |  |  |  |